- LMG01_04

- Cruise Data Report

10 April – 5 June 2001

# Table of Contents

Table of Contents	3

Introduction	1

Archive Data Extraction	2

Distribution Contents	3

ADCP	3

CTD	3

Maps	3

Mocness Data Archive	4

PUV – GUV	5

Isobar Charts	6

Data and Science Report	6

LMG JGOFS Data Products	6

RVDAS	7

Data File Names and Structures	7

XBT	13

XCTD	13

Processing Notes	13

LMG01_04 Sensors	14

Additonal information about Shipboard Sensors	14

LMG01_04 CTD Sensors:	15

Acquisition Problems and Events	16

# Introduction

The LMG data acquisition systems continuously log data from a suite of instrumentation throughout the cruise.  This document describes the format of that data and its location on the distribution CDs.  It also contains important information which may affect how this data is processed such as instrument failures or other known problems with acquisition.

The data collected during this cruise is distributed on a CD-ROM written in ISO9660 level-1 format.  This data format has very strict requirements on filenames and organization.  However, it is readable by virtually every computing platform.

The data is contained in Unix tar archives called lmgjgof1.tar, lmguw.tar, and lmgnav.tar.  All of the data has been compressed using Unix “gzip” compression.  Gzipped files have a “.gz” extension.  Tools are available on all platforms for uncompressing and de-archiving these formats.  On Macintosh, Stuffit Expander with DropStuff will open a tar archive and uncompress gzipped and Unix compressed files.  For Windows9X, WinZip, a shareware utility included on this CD (remember, it is shareware) will open these files.

IMPORTANT:  Read the last section in this document, Acquisition Problems and Events, for important information that may affect the processing of this data.

# Archive Data Extraction

It is often useful to know exactly how an archive was produced when expanding its contents.  Tar files were created using the following commands:

tar cvLf archive-file files-to-be-archived

To create a list of the files in the archive:

tar tvf  archive-file  > contents.list

To extract the files from the archive:

tar xvf  archive-file file(s)-to-extract

G-zipped files will have a “.gz” extension on the filename. These files can be decompressed after de-archiving, using:

gunzip filename.gz

The directories in the archive are structured in the following manner:

| LMG01_04/adcp/pingdata.zip LMG01_04/adcp/Gentoo.zip LMG01_04/calibration/calibration.zip LMG01_04/ctd/plots/plots.zip /Process/process.zip /Raw/raw.zip /Seasave/seasoft.zip LMG01_04/Isobars/isobars.zip LMG01_04/JGOF/lmgjgof.tar LMG01_04/Maps LMG01_04/MOCNESS/Moc1.zip /Moc10.zip /Moccnfg instrument.coeff report/ report.doc report.html report.txt LMG01_04/PUV-GUV/ Csvfiles.zip Graphs.zip Logfiles.zip | LMG01_04/rvdas/lmguw.tar DISK 1 LMG01_04/rvdas/lmgnav.tar DISK 2 LMG01_04/XBT/LMG01_04s.zip /LMG01_04e.arj /LMG01_04n.arj /LMG01_04s.arj LMG01_04/XCTD/XCTD.zip utilities/ Winzip Aladdin Stuffit Expander |
| --- | --- |

# Distribution Contents

## ADCP

- Adcp/

The ADCP data set is broken up into files representing 24 hours of data collection.  The files are contained in the pingdata.zip archive and are named pingdata.xxx (xxx representing a sequential day number starting from 000).  Note that these extensions do NOT represent Julian day numbers.  Please refer to the file’s creation date.

Additionally, Gentoo data is contained in Gentoo.zip. The files are named ADCP2001.xxx (xxx representing the julian day number).

Some ADCP data is also transmitted to RVDAS.  East and North vectors for ship’s speed relative to the reference layer and ship’s heading are archived in the navigational data section of RVDAS.

## CTD

- Ctd/

| process/ (post processed datafiles) raw/ (raw datafiles) plots/ (up & down trace postscript plots) seasoft4.242/ (application for processing ctd data) |  |
| --- | --- |

All CTD data acquired on the cruise can be found within this ctd directory. The data for each station consists of four files archived in the RAW folder. Each filename consists of the station number and extension: either .dat, .con, .bl, or .hdr. The type of data in each file is listed below.

.dat	Actual cast data in SeaBird format.

.bl	Information pertaining to the water sampling bottles for each cast.

.hdr	ASCII “header” containing user specific information for each cast.

.con	Configuration file used during each cast.

.nav	Position and time stamp generated at the beginning, bottom and end of the cast.

The Data collected was post-processed using seasoft standard processing and is stored in the PROCESS directory.

.cnv	‘Converted’ engineering unit data file

b.cnv	‘Converted’ engineering unit data file with data averaged into pressure bins

.ros	Scans marked with the bottle fire confirmation bit

.btl	A summary of the data in the .ros file

.asc	The data portion of the .cnv converted data file written in ascii

.hdr	Header file

Within the ctd directory, the setup directory contains SeaSoft v4.242, which can be used to replay, display, or manipulate the data.

Processing Note: SEASOFT modules search the current directory for DSP, PLT, and CFG files.  SEASOFT modules search the ‘input data path’ for CON, HEX, DAT, and CNV files.  One exception is SEACON which searches the current directory for CON files.

For more information and updated software visit the web site at www.seabird.com.

## Maps

JPG cruise track plots have been produced for this cruise: station1.jpg thru station5.jpg and final maps, with and without CTD points plotted.  Additionally, the cruise track file (LMG01_04.trk) is also included and contains the latitude and longitude in one minute intervals.

## Mocness Data Archive

A total of 23 MOC10 and 18 MOC1 hauls were made during this cruise. Each haul’s data is recorded in three separate and different text files and one graphics file. The format of each of these is summarized below. “M01” prefix = 1m2 MOCNESS, “M10” prefix = 10m2 MOCNESS.

Processed filename *.pro

This file contains the processed data for each tow, written in ASCII to disk in a simple configuration, which consists of:

Rows 1-5		header information about sensors and the tow

Row 6	the column headings (discussed below): time    pres  echo    temp   theta   sal     sigma  angle flow   hzvel    vtvel    vol     net  fluor   ptran    oxycurrent   oxytemp   oxygen     lat       lon

Succeeding rows	data for each column heading with each value separated by 2 spaces.

time    		Julian day, hours and minutes expressed as decimal

pres  		depth (m)

temp   		temperature degrees C

theta   		potential temperature, formulae from Fofonoff and Millard 1983

sal     		salinity ppt

sigma  		potential density, formulae from Fofonoff and Millard 1983

angle 		angle 0-90

flow   		flow counts 0000-9999

hzvel    		horizontal velocity (knots)

vtvel  		vertical velocity	(m/min)

vol   		seawater volume filtered  (m3)

net  		net number

fluor 		fluorescence (0-5 volts)

ptran    		extinction coefficient (0-5 volts)

oxycurrent 	0-5 volts

oxytemp   	0-5 volts internal to the probe

oxygen   	dissolved oxygen (ml/l)

lat 		latitude decimal degrees

lon		longitude decimal degrees

Raw filename *.raw

This file contains the raw data from the underwater unit. These “*.raw” files can be used in the playback mode to re-process the data (see page 33 MOCNESS instructions), and also serve as the backup in case there are problems with the processed data file.

For each tow, data is written (as it appears in the acquisition window) in ASCII to disk in a simple configuration, which consists of:

Rows 1-5		header information about sensors and the tow

Succeeding rows	raw data string formatted thusly: #MN- N1 N2 AA FFFF PPPPP PTPT TTTTTT CCCCCC BBB $GPGLL The fields are as follows and are discussed individually below.

- N1		net count, counts of left response switch, 00-99

- N2		net count, counts of right response switch, 00-99

- AA		net angle in degrees, 00-99

- FFFF	flow counts, 0000-9999. Reset to 0000 with every net response or increment net # button command

- PPPPP	pressure value, converted to decibars by the deck computer

- PTPT	a decimal number derived from the temperature sensor in the pressure sensor which is used to improve the pressure calibration

- TTTTTT	averaged SeaBird temperature period. The frequency from the SeaBird temperature sensor is measured and processed in the NCU to generate the value TTTTTT, a 6-digit decimal number. This number is scaled as follows:

(frequency of the temp. sensor)=K/(TTTTTT)

where K=1,258,291,200. Software in the deck computer uses this frequency value along with the individual sensor’s calibration file to calculate temperature.

-    CCCCCC	averaged SeaBird conductivity period, handled exactly like the temperature frequency and the same scaling factor should be used

- BBB	battery voltage (divide value by 10)

- $GPGLL	latitude and longitude in decimal degrees

Tab filename *.tab

The statistical summary for a given *.pro file. For each net, the following parameters are included:

pmin,pmax,pavg 		minimum,maximum and average depth of net

tmin,tmax,tavg		minimum,maximum and average temperature of net

smin,smax,savg		minimum,maximum and average salinity of net

amin,amax,aavg		minimum,maximum and average angle of net

spmin,spmax,spavg		minimum,maximum and average horizontal velocity (kt) of net

armin,armax,aravg		minimum,maximum and average vertical velocity (m/sec) of net

#obs				total number of observations while net was open

vol				total seawater volume filtered for net

Captured screens, filenames *.jpg

This file is a jpeg image of the acquisition software captured at any point during a MOCNESS “flight”. The files on this CD reflect the acquisition window at the end of each “flight”, immediately prior to ending acquisition.

## PUV – GUV

- PUV-GUV/

All GUV files, both binary and ASCII, have been zipped and placed in zip archives called csvfiles.zip and logfiles.zip respectively.

The GUV files were converted to CSV (Comma Separated Value) format using the Biospherical Instruments’ PUVPROF program.

The columnar format of the CSV file is listed below:

| Field | PUV Data |  | Field | GUV Data |
| --- | --- | --- | --- | --- |
| 1 | Depth |  | 1 | Depth |
| 2 | 308 U |  | 2 | 306 U |
| 3 | 320 U |  | 3 | 320 U |
| 4 | Temp U |  | 4 | Temp U |
| 5 | MATFL |  | 5 | MATFL |
| 6 | 340 U |  | 6 | 340 U |
| 7 | 380 U |  | 7 | 380 U |
| 8 | PAR U |  | 8 | PAR U |
| 9 | Temp U |  | 9 | Temp S |
| 10 | 308 S |  | 10 | 300 S |
| 11 | Batt S |  | 11 | Batt S |
| 12 | 320 |  | 12 | 320 S |
| 13 | 340 |  | 13 | 340 S |
| 14 | 380 |  | 14 | 380 S |
| 15 | Gnd |  | 15 | Gnd S |
| 16 | PAR |  | 16 | PAR S |
| 17 | K308 |  |  |  |
| 18 | K320 |  |  |  |
| 19 | K340 |  |  |  |
| 20 | K380 |  |  |  |
| 21 | Kpar |  |  |  |
| 22 | Chl |  |  |  |
| 23 | Prod |  |  |  |

## Isobar Charts

- Isobars/

Analysis of mean sea level pressure from the National Center for Environmental Prediction's Medium Range Forecast Model.  Updated every 12 hours.

## Data and Science Report

- Report/

Copies of this report in MS Word, HTML, and text formats.  The weekly science reports in text format may be included upon request of the Principal Investigator.

## LMG JGOFS Data Products

- JGOF/

The JGOFS data set consists of a single file produced each day named jg<julian_day>.dat.gz where <julian_day> is the day the data was acquired. The “.gz” extension indicates that the individual files are compressed before archiving. The daily file consists of 22 separate columnar fields in text format, which are described below. The JGOFS data set is obtained primarily by applying calibrations to raw data and decimating to whole minute intervals.  However, several fields are derived measurements from more than a single raw input.  Note: Null, unused, or unknown fields are filled with 9’s in the JGOFS data.

Additionally, 3 separate QC plots are generated daily by the ET using the JGOFS data set.  These plots include TSG and Bathymetry data, meteorological data, and navigation data.  The files are called ocean<julian_day>.ps, met<julian_day>.ps, and nav<julian_day>.ps respectively.

| Field | Data | Units |
| --- | --- | --- |
| 01 | GMT date | dd/mm/yy |
| 02 | GMT time | hh:mm:ss |
| 03 | PCOD latitude (negative is South) | Ddd.dddd |
| 04 | PCOD longitude (negative is West) | Ddd.dddd |
| 05 | Ships speed | Knots |
| 06 | GPS HDOP | - |
| 07 | Gyro Heading | Degrees (azimuth) |
| 08 | Course over ground | Degrees (azimuth) |
| 09 | Mast PAR | Einsteins/meters2 sec |
| 10 | Sea surface temperature | C |
| 11 | Not used | - |
| 12 | Sea surface salinity | PSU |
| 13 | Sea depth (uncorrected, calc. sw sound vel. 1500 m/s) | meters |
| 14 | True wind speed (port windbird) | meters/sec |
| 15 | True wind direction (port windbird) | degrees (azimuth) |
| 16 | Ambient air temperature | C |
| 17 | Relative humidity | % |
| 18 | Barometric pressure | mBars |
| 19 | Sea surface fluorometry | volts (0-5 FSO) |
| 20 | Not used | - |
| 21 | PSP | W/m2 |
| 22 | PIR | W/m2 |

## RVDAS

rvdas/

RVDAS (Research Vessel Data Acquisition System) was developed at Lamont-Doherty Earth Observatory of Columbia University and has been used on the R/V Maurice Ewing for several years.  It was adapted for use on the Nathaniel B. Palmer and her sister ship, the R/V Laurence M. Gould.

Below you will find detailed information on the data included. Be sure to read the “Significant Acquisition Events” section below for important information about data acquisition during this cruise.

#### Meteorological and Light Data

| Measurement | File ID | Collect. Status | Rate | Instrument |
| --- | --- | --- | --- | --- |
| Air Temperature | lmet | continuous | 1 sec | R. M. young 41372VC |
| Relative Humidity | lmet | continuous | 1 sec | R. M. young 41372VC |
| Wind Speed/Direction | lmet | continuous | 1 sec | R. M. young 5106 |
| PIR (LW radiation) | lmet | continuous | 1 sec | Eppley PIR |
| PSP (SW radiation) | lmet | continuous | 1 sec | Eppley PSP |
| Photosynthetically- Available Radiation | lmet | continuous | 1 sec | BSI QSR-240 |
| Barometer | lmet | continuous | 1 sec | R. M. young 61201 |

#### Navigational Data

| Measurement | File ID | Collect. Status | Rate | Instrument |
| --- | --- | --- | --- | --- |
| Attitude GPS | lash | continuous | 1 sec | Ashtec ADU-2 |
| P-Code GPS | lpcd | continuous | 1 sec | Trimble 20636-00SM |
| Gyro | lgyr | continuous | 0.2 sec | Yokogawa Gyroscope |
| Trimble GPS | tgps | continuous | 1 sec | NT200 |

#### Geophysical Data

| Measurement | File ID | Collect. Status | Rate | Instrument |
| --- | --- | --- | --- | --- |
| Bathymetry | lknu | variable | Varies | Knudsen 320B/R |

#### Oceanographic Data

| Measurement | File ID | Collect. Status | Rate | Instrument |
| --- | --- | --- | --- | --- |
| Salinity | ltsg | continuous | 15 sec | SeaBird 21 |
| Sea S Temperature | ltsg | continuous | 15 sec | SeaBird 3-01/S |
| Fluorometry (analog) | ltsg | continuous | 15 sec | Turner 10-AU-005 |
| ADCP | ladc | continuous | 1 sec | RD Instruments |

### Data File Names and Structures

RVDAS data is divided into two broad categories, Underway and Navigation.  The groups are abbreviated “uw” and “nav”. Thus, these two tar files, lmguw.tar and lmgnav.tar exist under the top-level rvdas directory. The instruments are broken down as shown. Each data file is g-zipped to save space on the distribution.  Not all data types are collected everyday or on every cruise.

RVDAS data files are named following the convention: LMG[FileID].dDDD.

- The FileID is a 4-character code representing the system being logged, for example: lmet (for meteorology)

- DDD is the Julian day of the data collection

| Underway Data | File ID |  | Navigation Data | File ID |
| --- | --- | --- | --- | --- |
| Meteorolical | lmet |  | Gyro Compass | lgyr |
| Knudsen | lknu |  | P-CODE GPS | lpcd |
| Thermosalinograph | ltsg |  | Ashtech ADU2 GPS | lash |
| ADCP | ladc |  | Trimble NT2100 GPS | tgps |
| Sound Velocity Probe | lsvp |  |  |  |

Data is received by the RVDAS system via RS-232 serial connections. The data files that comprise the rvdas data set are described below. A time tag is added to each line of data received and the data is written to disk.

YY+DDD:HH:MM:SS.SSS [data stream from instrument]

Where, YY: two-digit year, DDD: Julian Day, HH: 2 digit hours, MM: 2 digit minutes SS.SSS: seconds.  All times are UTC.

The delimiters used to separate fields in the raw data files are usually spaces and commas, but other delimiters are used (::, =, @) and occasionally there is no delimiter.  Care should be taken when reprocessing the data that the fields separations are clearly understood.  An example data

#### lknu

99+099:00:18:19.775 hf,305.2,lf,304.3

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | hf – high frequency flag (12 kHz) |  |
| 3 | high fequency depth | meters |
| 4 | lf – low frequency flag (3.5 kHz) |  |
| 5 | low frequency depth | meters |

#### lmet

- 00+040:00:00:00.928 16-10-97 00:01:19 005.53 005.53 005.69 145 141 005.15 005.12 005.15

- 146 148 004.4 004.4 004.4 94.07 94.06 94.07 0981.40 -0.023 00.292 00.315

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | Instrument date | dd-mm-yy |
| 3 | Instrument time | hh:mm:ss |
| 4 | Port wind speed (10 second running average) | m/s |
| 5 | Port wind speed (min for 24 hour cycle) | m/s |
| 6 | Port wind speed (max for 24 hour cycle) | m/s |
| 7 | Port wind direction | degrees |
| 8 | Port wind direction (standard deviation) | degrees |
| 9 | Starboard wind speed (10 second running average) | m/s |
| 10 | Starboard wind speed (min for 24 hour cycle) | m/s |
| 11 | Starboard wind speed (max for 24 hour cycle) | m/s |
| 12 | Starboard wind direction | degrees |
| 13 | Starboard wind direction (standard deviation) | degrees |
| 14 | Temperature | C |
| 15 | Temperature (min for 24 hour cycle) | C |
| 16 | Temperature (max for 24 hour cycle) | C |
| 17 | Relative humidity | % |
| 18 | Relative humidity (min for 24 hour cycle) | % |
| 19 | Relative humidity (max for 24 hour cycle) | % |
| 20 | Barometric pressure | millibars |
| 21 | PSP (long wave radiometer) | Volts |
| 22 | PIR (short wave radiometer) | Volts |
| 23 | PAR (photo-synthetically available radiation, 400 - 700 nm) | Volts |

#### svp1

00+348:01:59:52.128  1539.40

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | Sound velocity | m/s |

#### ltsg

00+040:00:00:06.153 6926 15.8827 0.2010 2.9218 3.239 4.334e-01

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | Scan number |  |
| 3 | Internal water temperature | C |
| 4 | Salinity | PSU |
| 5 | External water temperature | C |
| 6 | Transmissometer signal | Volts |
| 7 | Fluorometer signal (analog) | Volts |

#### ladc

00+019:23:59:59.099 $PUHAW,UVH,-1.48,-0.51,250.6

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | $PUHAW |  |
| 3 | UVH (E-W, N-S, Heading) |  |
| 4 | Ship Speed relative to reference layer1 velocity2, East vector | knots |
| 5 | Ship Speed relative to reference layer1 velocity2, North vector | knots |
| 6 | Ship heading | degrees |

1The reference layer is an average velocity measured in a number of depth “bins”. On the LMG, the bins are eight meters deep and bins 3-10 define the reference layer. Hence, the reference layer is the water column from 16-80 meters beneath the ship.

2The speed ouput is water velocity relative to the ship’s hull and is therefore opposite of the actual

movement of the ship. For example, if the ship’s heading is due north, the North/South reference layer

velocity is likely to be negative (southerly).

#### lash

##### ATTD: Attitude Data

01+081:00:00:00.806 $PASHR,ATT,345605.0,165.03,+001.86,-01.96,0.0018,0.0173,0*22

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $PASHR |  |
| 2 | ATT |  |
| 3 | GPS Time sec. of the week | seconds |
| 4 | heading (rel. to true North) | degrees |
| 5 | pitch | degrees |
| 6 | roll | degrees |
| 7 | Measuremnet RMS error | meters |
| 8 | Baseline RMS error | meters |
| 9 | attitude reset flag |  |

##### GGA: GPS Position Fix – Geoid

01+081:00:00:00.966 $GPGGA,235952.00,6051.7937,S,06030.2175,W,1,08,01.0,+00068,M,,M,,*79

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPGGA |  |
| 2 | $GPGGA, NEMA sentence label |  |
| 3 | UTC time at position | hhmmss.ss |
| 4 | Latitude | ddmm.mmm |
| 5 | North (N) or South (S) |  |
| 6 | Longitude | ddmm.mmm |
| 7 | East (E) or West (W) |  |
| 8 | GPS quality (1=GPS 2=DGPS) |  |
| 9 | Number of GPS satellites used |  |
| 10 | HDOP |  |
| 11 | Antenna Height | meters |
| 12 | M for Meters |  |
| 13 | Geoidal height | meters |
| 14 | M for meters |  |
| 15 | age of diff. GPS data | seconds |
| 16 | differential reference station ID |  |

#### lgyr

01+155:00:00:00.106 $IIVHW,001.2,T,,M,,N,,K*78

01+155:00:00:00.286 $HEHDT,001.2,T*2C

01+155:00:00:00.486 $HEROT,004.3,A*2C

01+155:00:00:00.665 $HCHDT,,T*07

01+155:00:00:00.885 $PASVW,06.0,A*1A

##### HDT: True Heading

01+083:00:00:02.893 $HEHDT,246.3,T*2C

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $HEHDT |  |
| 2 | Heading XXXXX = ddd.d | degrees |
| 3 | T flag for true heading, checksum |  |

##### ROT: Rate of Turn

01+083:00:00:03.093 $HEROT,-006.3,A*03

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $HEROT |  |
| 2 | Rate of turn | degrees/min |
| 3 | Status: A = data valid, checksum |  |

#### tgps

##### GGA: Global Positioning Fix Data

00+040:00:00:00.985 $GPGGA,000003,6139.961,S,05949.422,W,1,6,001.64,-00036,M,00000,M,,

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPGGA |  |
| 2 | Latitude in degrees with decimal minutes | ddmm.mmm |
| 3 | North (N) or South (S) |  |
| 4 | Longitude in degrees with decimal minutes | ddmm.mmm |
| 5 | East (E) or West (W) |  |
| 6 | GPS quality (1=GPS 2=DGPS) |  |
| 7 | Number of GPS satellites used |  |
| 8 | Horizontal dilution of precision (HDOP) |  |
| 9 | Antenna height above/below mean-sea-level (geoid) | meters |
| 10 | Units for antenna height (M = Meters) |  |
| 11 | Geoidal Separation1 |  |
| 12 | Units for Geoidal Separation (M = Meters) | meters |
| 13 | Age of differential GPS data, number of seconds since last SC104 Type 1 or 9 |  |
| 14 | Differental reference station ID |  |

1Geoidal Separation: the difference between the WGS-84 earth ellipsoid and mean-sea-level (geoid). A negative value represents mean-sea-level below ellipsoid.

##### GLL: Geographic Position – Latitiude/Logitude

00+040:00:00:00.065 $GPGLL,6139.96,S,05949.42,W,000002,A

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPGLL |  |
| 2 | Latitude | ddmm.mmm |
| 3 | North (N) or South (S) |  |
| 4 | Logitude | ddmm.mmm |
| 5 | East (E) or West (W) |  |
| 6 | UTC of position | hhmmss.ss |
| 7 | Status: A = Data Valid |  |

##### VTG: Track Made Good and Speed over Ground

00+040:00:00:00.213 $GPVTG,161,T,149,M,009.6,N,017.8,K

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPVTG |  |
| 2 | Track, degrees true | degrees |
| 3 | T flag for True |  |
| 4 | Track, degrees magnetic | degrees |
| 5 | M flag for Magnetic |  |
| 6 | Speed over Ground | knots |
| 7 | N flag for Knots |  |
| 8 | Speed over Ground | kmhr |
| 9 | K flag for km/hr |  |

##### VHW: Speed Through Water and Heading

00+040:00:00:00.212 $GPVHW,246,T,234,M,012.3,N,022.8,K

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPVHW |  |
| 2 | Heading, degrees True | degrees |
| 3 | T flag for True |  |
| 4 | Heading, degrees Magnetic | degrees |
| 5 | M flag for Magnetic |  |
| 6 | Speed through water | knots |
| 7 | N flag for Knots |  |
| 8 | Speed through water | km/hr |
| 9 | K flag for km/hr |  |

##### ZDA: Time and Date

00+040:00:00:00.285 $GPZDA,000002,09,02,2000,00,00

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPZDA |  |
| 2 | UTC time | hhmmss.ss |
| 3 | Day: 01 – 31 | dd |
| 4 | Month: 01 – 12 | mm |
| 5 | Year | yyy |
| 6 | Local time zone description1, 00 +/-13 hrs |  |
| 7 | Local time zone minutes description, same sign as local hours |  |

1Zone description is the number of whole hours added to local time to obtain GMT, values are negative for East longitudes.

##### BWC: Bearing and Distance to Waypoint

00+040:00:00:00.865 $GPBWC,000003,6209.70,S,05824.00,W,127.2,T,115.3,M,050.1,N,014

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPBWC |  |
| 2 | UTC of bearing | hhmmss.ss |
| 3 | Destination waypoint latitude in degrees, decimal minutes | ddmm.mmm |
| 4 | Hemisphere Flag: N or S |  |
| 5 | Destination waypoint longitude in degrees, decimal minutes | ddmm.mmm |
| 6 | Hemisphere Flag: E or W |  |
| 7 | Bearing, degrees true | degrees |
| 8 | T flag for True |  |
| 9 | Bearing, degrees magnetic | degrees |
| 10 | M flag for Magnetic |  |
| 11 | Distance to waypoint in nautical miles | nm |
| 12 | N flag for Nautical Miles |  |
| 13 | Waypoint ID |  |

#### lpcd

##### GGA: GPS Position Fix – Geoid/Ellipsoid

00+019:23:59:59.301 $GPGGA,235958.409,6849.6944,S,13712.8472,W,1,06,1.2,092.4,M,047.3,M,,*67

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPGGA |  |
| 2 | UTC time at position | hhmmss.sss |
| 3 | Latitude | ddmm.mmm |
| 4 | North (N) or South (S) |  |
| 5 | Longitude | ddmm.mmm |
| 6 | East (E) or West (W) |  |
| 7 | GPS quality (1=GPS 2=DGPS 3=P-CODE) |  |
| 8 | Number of GPS satellites used |  |
| 9 | HDOP |  |
| 10 | Antenna Height | meters |
| 11 | M for Meters |  |
| 12 | Geoidal height | meters |
| 13 | M for meters |  |
| 14 | Null field |  |
| 15 | Checksum |  |

##### GLL: GPS Latitude/Longitude

00+019:23:59:59.381 $GPGLL,6849.6944,S,13712.8472,W,235958.409,A*35

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPGLL |  |
| 2 | Latitude | degrees |
| 3 | North or South |  |
| 4 | Longitude | degrees |
| 5 | East or West |  |
| 6 | UTC of position | hhmmss.sss |
| 7 | staus of data (A = valid) |  |
| 8 | Checksum |  |

##### VTG: GPS Track and Ground Speed

00+019:23:59:59.382 $GPVTG,238.7,T,182.3,M,001.8,N,003.3,K*41

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPVTG |  |
| 2 | Heading | degrees |
| 3 | degrees True (T) |  |
| 4 | Heading | degrees |
| 5 | degrees magnetic (M) |  |
| 6 | Ship speed | knots |
| 7 | N = knots |  |
| 8 | Speed | km/hr |
| 9 | Checksum |  |

## XBT

- XBT/

During the cruise Expendable Bathythermographs were used to obtain water column temperature profiles.  The data files from these launches are included.

## XCTD

- XCTD/

During the cruise Expendable CTDs were used to obtain water column temperature profiles.  The data files from these launches are included.

## Processing Notes

Refer to the instrument.coeff file, included in the root level of the data distribution, along with the specific instrument calibration sheets, found at the end of this report, for information on how the RVDAS data was processed.

# LMG01_04 Sensors

The most recent calibration sheets for CTD Instrumentation, Mast Radiometer Instrumenation, and TSG Instrumenation have been included on the CD in the Cal/ directory.  These are scanned image files of the hard copy calibration sheets.  A key to match filenames to instruments is shown below.

| TSG Primary Temperature (Internal) | Tsg_temp.jpg |
| --- | --- |
| TSG Secondary temperature (Sea Water Intake) | Rem_temp.gif |
| TSG Conductivity | Tsg_Cond.jpg |
| Underway Transmissometer | UW_Trans.gif |
| CTD Primary Temperature | Pri_Temp.jpg |
| CTD Secondary Temperature | Sec_Temp.jpg |
| CTD Primary Conductivity | Pri_Cond.jpg |
| CTD Secondary Conductivity | Sec_Cond.jpg |
| CTD Primary Pump | Pri_Pump.jpg |
| CTD Secondary Pump | Sec_Pump.jpg |
| CTD PAR | CTD_Par.jpg |
| CTD Transmissometer | CTD_Trans.jpg |
| CTD Dissolved Oxygen | CTD_Ox.jpg |
| CTD Fluorometer | CTD_fluor.jpg |
| CTD Fish | CTD_fish.jpg |
| Mast PAR | Mast_PAR.jpg |
| Mast PSP | Mast_PSP.jpg |
| Mast PIR | Mast_PIR.jpg |
| MOC1 Depth | MOC_149.jpg. |
| MOC10 Depth- initial | MOC_139.txt |
| MOC10 Depth- after 139 failed | MOC_148.jpg |
| Port Anemometer | Portanem.jpg |
| Starboard Anemometer | Stbdanem.jpg |
| Barometer | Barometr.jpg |
| Humidity/Wet Temperature | Humidity.jpg |
| GUV | GUVa.jpg GUVb.jpg |

## Additonal information about Shipboard Sensors

| Sensor | Description | Serial # | Cal. Date | Status |
| --- | --- | --- | --- | --- |
| Port Anemometer | R.M. Young 105106 | VM35061 | 04/15/01 | collect |
| Stbd Anemometer | R.M. Young 105106 | VM28393 | 04/15/01 | collect |
| Barometer | R.M. Young 61201 | BP00873 | 11/30/99 | collect |
| Humidity/Wet Temp | R.M. Young 41372VC | 4018 | 11/07/01 | collect |
| Mast PAR | BSI QSR-240 | 6393 | 03/22/99 |  |
| Pyranometer | Eppley PSP | 28933F3 | 07/26/00 | collect |
| Pyrgeometer | Eppley PIR | 28903F3 | 07/26/00 | collect |
| GUV | Biospherical GUV-511 | 9266 | 11/08/00 |  |
| TSG | SeaBird SBE21 | 2110410-1789 | 02/11/00 | collect |
| TSG Remote Temp | SeaBird 3-01/S | 1620 | 1/4/00 | collect |
| Fluorometer | Turner 10-AU-005 Lamp: daylight 10-045, reference filter: 10-052, emission filter: 10-051, excitation filter: 10-050. | 6046RTD |  | collect |
| Transmissometer | WET Labs 9707017 | CST-168R | 11/30/00 | collect |
| P-Code GPS | Trimble 20636-00 (SM) | 220035265 | 04/01/01 | PCD/CIV |
| Bathymetry | Knudsen 320B/R | K97715 |  |  |

## LMG01_04 CTD Sensors:

| Sensor | Description | Serial # | Cal. Date | Status |
| --- | --- | --- | --- | --- |
| CTD Fish | Sea-Bird 9Plus-3400m | 91480 | 6/8/99 |  |
| CTD Deck Unit | Sea-Bird 11Plus | 111411 |  |  |
| CTD Deck Unit- spare | Sea-Bird 11Plus | 11P8908-0288 |  |  |
| Prim. Temp. Sensor | Sea-Bird 3-02/F | 2444 | 1/30/01 |  |
| Sec. Temp. Sensor | Sea-Bird 3-02/F | 1542 | 1/19/01 |  |
| Prim. Cond. Sensor | Sea-Bird 4-02/0 | 692 | 8/22/00 |  |
| Sec. Cond. Sensor | Sea-Bird 4C | 2048 | 8/22/00 |  |
| Diss. Oxygen Sensor | Sea-Bird 13-02-B | 130412 | 5/4/00 |  |
| Fluorometer | Chelsea model Mk III Aquatracka | 088211 | 3/3/99 |  |
| Transmissometer | Wet-Labs C-Star 25cm | CST-407DR | 11/16/00 |  |
| PAR | Biospherical QSP-200L4S | 4470 | 7/31/00 |  |

# Acquisition Problems and Events

This section lists all known problems with acquisition during this cruise including instrument failures, data acquisition system failures and any other factor affecting this data set. The format is jjj:hh:mm (jjj is julian day, hh is hour, and mm is minute).  All times are in GMT.

- 110	23:00 start cruise

- 110	23:25 power on to SONAR

- 111	01:00 SONAR off

- 111	??SONAR on

- 111	12:00 ADCP on

- 111	12:10 Discovered that Ashtech is not working- shut down ADCP

- 111	12:45 Cyclone locked up- rebooted

- 111	13:25 Ashtech reset- came back up

- 111	13:36 ADCP re-started

- 111	13:38 Noticed that SVP did not re-initialize when Cyclone re-booted- restarted it.

- 112	13:40 Turned off bottom tracking on ADCP

- 112	13:54 Deployed drifter buoy 30053

- 112	~08:45 Started XBT run

- 113	04:47 Deployed drifter buoy 30052

- 113	05:15 Started TSG-58 04.2 S Lat 62 43.2 W Lon- (at 200 mi. limit)
First 7 scans had no water running- oops

- 113	22:43 started GUV after resetting Calibration factors

- 114	12:55 Turned Bottom tracking on for ADCP

- 114	~13:20 Arrive Maxwell Bay, King George Island

- 114	19:19 shutdown Knudsen for Biofish tests

- 114	19:21 shutdown ADCP for Biofish testing

- 114	19:38 Re started Knudsen

- 114	21:48 Ships power off- All systems seem OK except TSG

- 114	22:03 TSG back on watch data- due to pump shutdown

- 114	23:24 Ships power off- All systems seem OK except TSG

- 114	23:28 TSG back on watch data- due to pump shutdown

- 115	06:30-08:45 Knudsen lost bottom due to shallower water than TX blank

- 115	10:15-12:45 Knudsen lost bottom due to shallower water than TX blank

- 116	00:30 Switched Knudsen from 3.5kHz to 12 kHz for noise reasons

- 116	00:56 Stopped ADCP computer to backup files

- 116	01:08 Stopped logging on Gentoo

- 116	01:27 Re-started ADCP

- 117	Changed SONAR back to 3.5kHz for JGOFs- noticed in QC scripts that changing the SONAR not only causes the JGOF’s to display 9999 for depth, but also causes seawater temp to go to zero? The data is still being correctly collected by the TSG and written to /data/not-public/logger/uw/LMGltsg.day, but for some reason JGOF’s displays 0.

- 117	11:41 Shutdown TSG – arrived at Palmer Station

- 117	11:44 Shutdown ADCP

- 117	11:52 Shutdown SONAR

- 117	19:30 shutdown GUV for testing

- 117	19:45 restarted GUV

- 117	20:00 cleaned GUV lens, PIR, PSP and PAR sensors

- 118	17:10 Re-start SONAR- depart palmer Station

- 118	17:13 Re-start ADCP

- 118	17:20 Re-started TSG

- 119	11:28 Stopped logging on GUV to double check values-set time to 10 minute intervals

- 119	11:31 Restarted logging on GUV

- 120	22:30 Stopped Knudsen for ADCP survey termination

- 120	22:48 Restarted Knudsen

- 120	23:56 Stopped ADCP for data backup

- 121	00:14 Restarted ADCP

- 121	00:19 Stopped GUV deck unit for data backup

- 121	00:29 Restarted GUV

- 122	18:06 Stopped ADCP to copy files

- 122	18:09 Restarted ADCP

- 122	Problem with 12 hour Isobars resolved. Now receiving 2X per day.

- 126	17:08 Stopped ADCP to copy files

- 126	17:37 Restarted ADCP

- 128	00:59 Stopped ADCP for HTI trouble shooting.

- 128	01:04 Restarted ADCP, Stopped Knudsen SONAR

- 128	01:06 Restarted Knudsen SONAR

- 128	13:04 Stopped ADCP to copy files

- 128	13:14 Restarted ADCP

- 128	23:53 Stopped GUV logging to back up files

- 129	00:10 Restarted GUV logging.

- 129	01:23 Changed GUV logging interval from 10 minutes to 1 minute.

- 129	18:23 Stopped ADCP to copy files

- 129	18:39 Restarted ADCP

- 131	17:58 Stopped ADCP to copy files

- 131	18:13 Restarted ADCP

- 131	~19:20 Cleaned PIR, PAR, PSP, and GUV on mast.

- 134	00:38 Stopped GUV logging to check data.

- 134	00:40 Restarted GUV logging.

- 134	~10:00 Port Windbird speed and direction data stream died. Believed to be the electronics module.

- 134	16:29 Stopped ADCP to copy files

- 134	16:36 Restarted ADCP

- 134	15:03 TSG logging shut down by an unknown cause or person.

- 134	15:00 GUV logging was shut down by unknown cause or person.

- 134	16:50 I restarted the TSG upon noticing that it was off.

- 134	16:59 Shut down MET to change port wind module.

- 134	17:47 restarted MET, wind direction is now working but wind velocity is still bad. PAR came loose during replacement. Re-wired, but possible bad readings.

- 134	18:19 Shut down MET system to check wiring on PAR. Output is 0.000V

- 134	19:05 Restarted MET system.

- 134	19:30- 19:53 pulled PAR signal and gnd wires from back of MET module. Looked at voltages and found 0.9V DC, which the Manufacturer cites as the level for background noise. Repeatedly removed and re-installed the wires during this time. Unfortunately it is now too dark to tell if 0.000V is accurate or not. Will watch reading tomorrow.

- 134	22:01 Noticed that Port wind speed seems to be working again. Bridge said it happened about 21:00. PAR OK as well.

- 134	23:59 TSG and MET off. Today I discovered that several calibration constants reflected previously installed sensors. On Cyclone, the /usr/local/packages/rvdas/config/instrument.coeff file had not been updated when the mast sensors were installed. Specifically, I changed the PIR coefficient from 3.86 to 3.57, the PSP coefficient from 8.85 to 8.23 and the transmissometer Vdark from 0.062 to 0.058 and the Vref from 4.781 to 4.801. 
Additionally, I changed the calibration coefficients for the underway TSG.
Specifically, I changed the Conductivity factors from 
the values for 1577: 				to the values for 1789:

g = -3.94145559e+00				g = -4.04904711e+00

h = 4.69434689e-01				h = 4.83917250e-01

i = 1.49060769e-03				i = 9.06133798e-04

j = -4.31070264e-05				j = -1.37490237e-05

The primary temperature was already correct, but the secondary temp had to be updated from 1619:				 to 1620:

g = 4.79514050e-03				g = 4.79485872e-03

h = 6.68331805e-04				h = 6.68466216e-04

i = 2.95597429e-05				i = 2.97709921e-05

j = 2.79206188e-06				j = 2.84551361e-06

I saved the new config file as lmg0104b.con; the old one being lmg0104.con.

- 135	00:29 Restart RVDAS135

- 135	00:31 SV Probe stopped working.

- 135	01:05 restarted the SV Probe

- 137	00:58 Noticed that GUV logging had been shut off. Also noticed that the date was two days behind. I changed the date from 5/15/01 to 5/17/01. Looked for signs that the date had been off previously, but found none.  I believe that the date was not incremented during the period that the GUV was off.

- 137		01:00 Restarted logging on GUV

- 137		13:33 Turned off H2O to UW transmissometer to clean lenses

- 137		13:45 Turned on H2O to transmissometer- big jump in Transmission %.

- 137		13:49 Turned off H2O to UW flourometer to clean

- 137		13:51 Turned on H2O to flourometer

- 137		19:15 Cleaned CTD transmissometer lenses

- 139		02:52 Ashtech stopped logging for unknown reasons

- 139		08:00 CTD deck unit #111411 failed at start of cast # 43. Replaced with spare. #11P8908-0288

- 139		23:25 Turned off GUV logging to backup files

- 139		23:27 Turned GUV logging back on.

- 139		23:32 Changed PUV logging back to minutely. It had reverted to every 10 minutes when it was shut down.

- 140		~02:00 to 03:19 RVDAS shutdown and restarted several times in an attempt to fix the Ashtech.

- 140		03:19 Ashtech fixed – re-started logging.

- 142	16:57 Cleaned PAR, PIR, PSP, GUV

- 144	00:00 Turned off GUV logging to backup files

- 144	00:08 Turned on GUV logging

- 144	15:00 CTD deck unit again failed due to buffer overrun. Problem traced to SPAR.

- 144	22:16 Stopped ADCP for data backup

- 144	Restarted ADCP

- 145	02:30 Ashtech failed. This time only for 2 minutes before it recovered.

- 145	13:00 Gentoo locked up and had to be restarted. No ADCP logging.

- 145	13:26 Gentoo restarted and ADCP logging recommenced.

- 148	16:00 Stopped logging on ADCP to back up files

- 148	16:15 Restarted logging.

- 151	04:35 PUV Off- Arrival at Palmer Station

- 151	04:37 TSG off

- 151	04:40 ADCP off

- 151	12:50 SONAR off

- 151	17:09 SONAR on

- 151	17:54 ADCP on

- 151	Turned TSG on and noticed that the transmissometer readings were low, (in the 30's). I had cleaned the lenses during the stop at Palmer so I thought maybe there was soap left on the lenses or something. I re-cleaned them and was still getting low and variable readings. I then removed the cable and cleaned the contacts. After re- connecting the readings were still low but more believable. (mid to high 70s)

- 151	18:36 Finished cleaning contacts and re-connected cable to Transmissometer.

- 151	18:46 ADCP was not logging to RVDAS system so I shut it down.

- 151	18:59  Found that problem was due to incorrect Ue4.cnf file sent by Teri Chereskin. She had the speed logging turned off on COM2. ADCP restarted.

- 151	19:33 ADCP stopped for Meng Zhou to look at files

- 151	20:15 Re-started ADCP

- 152	23:40 Shutdown TSG at 200 mile limit

- 152	22:58 ADCP Bottom tracking back on

- 155	02:07 Ashtech failed again

- 155	02:43 Ashtech restarted

- 155	18:11 Stopped ADCP at mouth of Straight of Magellan

- 155	18:39 Stopped all data collection and ended cruise.