- LMG01_06

- Cruise Data Report

# 21 July – 1 September 2001

# Table of Contents

Introduction	1

Archive Data Extraction	2

Distribution Contents	3

ADCP	3

CTD	3

Maps	3

Mocness Data Archive	3

PUV – GUV	5

Isobar Charts	6

Data and Science Report	Error! Bookmark not defined.

LMG JGOFS Data Products	Error! Bookmark not defined.

RVDAS	6

Data File Names and Structures	7

XBT	13

Processing Notes	13

LMG01_06 Sensors	14

Additonal information about Shipboard Sensors	14

LMG01_06 CTD Sensors:	15

Acquisition Problems and Events	16

# Introduction

The LMG data acquisition systems continuously log data from a suite of instrumentation throughout the cruise.  This document describes the format of that data and its location on the distribution CDs.  It also contains important information which may affect how this data is processed such as instrument failures or other known problems with acquisition.

The data collected during this cruise is distributed on a CD-ROM written in ISO9660 level-1 format.  This data format has very strict requirements on filenames and organization.  However, it is readable by virtually every computing platform.

The data is contained in Unix tar archives called lmgjgof1.tar, lmguw.tar, and lmgnav.tar.  All of the data has been compressed using Unix “gzip” compression.  Gzipped files have a “.gz” extension.  Tools are available on all platforms for uncompressing and de-archiving these formats.  On Macintosh, Stuffit Expander with DropStuff will open a tar archive and uncompress gzipped and Unix compressed files.  For Windows9X, WinZip, a shareware utility included on this CD (remember, it is shareware) will open these files.

IMPORTANT:  Read the last section in this document, Acquisition Problems and Events, for important information that may affect the processing of this data.

# Archive Data Extraction

It is often useful to know exactly how an archive was produced when expanding its contents.  Tar files were created using the following commands:

tar cvLf archive-file files-to-be-archived

To create a list of the files in the archive:

tar tvf  archive-file  > contents.list

To extract the files from the archive:

tar xvf  archive-file file(s)-to-extract

G-zipped files will have a “.gz” extension on the filename. These files can be decompressed after de-archiving, using:

gunzip filename.gz

The directories in the archive are structured in the following manner:

| ADCP/ Gentoo.zip Pingdata.zip Cals/ Cals.zip README.zip CTD/ Data.zip Displays.zip Seasoft.zip Bottles.zip GUV/ CSV.zip Log.zip PARPlots.zip Isobars/ Isobars.zip Ice/ Ice.zip MOCNESS/ Data.zip Net_Stats.zip Screenshots.zip Sensors.zip | RVDAS/ Lmgjgof.tar Lmgnav.tar Lmguw.tar Instrument.coeff Report/ Cruise Data Report.zip Map/ GLOBEC_II.jpg QFAX/ QFAX.zip XBT/ EDF.zip RDF.zip XBTPlots.zip |
| --- | --- |

# Distribution Contents

## ADCP

#### Adcp/

The ADCP data set is broken up into files representing 24 hours of data collection.  The files are contained in the pingdata.zip archive and are named pingdata.xxx (xxx representing a sequential day number starting from 000).  Note that these extensions do NOT represent Julian day numbers.  Please refer to the file’s creation date.

Additionally, Gentoo data is contained in Gentoo.zip. The files are named ADCP2001.xxx (xxx representing the julian day number).

Some ADCP data is also transmitted to RVDAS.  East and North vectors for ship’s speed relative to the reference layer and ship’s heading are archived in the navigational data section of RVDAS.

## CTD

Ctd/

/Data contains the *.dat. *.hdr, *.bl, and *.con raw data files for each cast. The first cast filename has prefix "13", not conforming to the rest of the filenames. Its name should have been "208_1…" but it was not changed since filename changing can interfere with postprocessing programs.

All CTD data acquired on the cruise can be found within this ctd directory. The data for each station consists of four files archived in the RAW folder. Each filename consists of the station number and extension: either .dat, .con, .bl, or .hdr. The type of data in each file is listed below.

.dat	Actual cast data in SeaBird format.

.bl	Information pertaining to the water sampling bottles for each cast.

.hdr	ASCII “header” containing user specific information for each cast.

.con	Configuration file used during each cast.

.nav	Position and time stamp generated at the beginning, bottom and end of the cast.

The Data collected was not post-processed, except to produce the bottle-summary files found in Bottles.zip. For this purpose, the Seasoft program DATCNV was followed by ROSSUM. You must use the Seasoft software supplied in Seasoft.zip to convert the raw data files into readable form.

.

Within the ctd directory, the setup directory contains SeaSoft v4.242, which can be used to replay, display, or manipulate the data.

Processing Note: SEASOFT modules search the current directory for DSP, PLT, and CFG files.  SEASOFT modules search the ‘input data path’ for CON, HEX, DAT, and CNV files.  One exception is SEACON which searches the current directory for CON files.

For more information and updated software visit the web site at www.seabird.com.

## Maps

A cruise map was produced showing the ship track and various events in Marguerite Bay. Since it was produced first in GMT, which outputs postscript format, and then converted by GSview to JPEG for further work in Adobe Photoshop, it is supplied as a JPEG file.

Please do not modify this map for publication or display or use it in any way for commercial purposes without permission from the cartographer, Pete Martin (martinp@geo.orst.edu).

## Mocness Data Archive

A total of 5 MOC1 tows were done during this cruise. The raw data records are at 4-second intervals, including temperature, conductivity, transmissivity, fluorometer voltage, flow count, and ship GPS position. The Net_Stats summarize the conditions during each net open. The screenshots provide a graphic of the tow.

#### Processed filename *.pro

This file contains the processed data for each tow, written in ASCII to disk in a simple configuration, which consists of:

Rows 1-5		header information about sensors and the tow

Row 6	the column headings (discussed below): time    pres  echo    temp   theta   sal     sigma  angle flow   hzvel    vtvel    vol     net  fluor   ptran    oxycurrent   oxytemp   oxygen     lat       lon

Succeeding rows	data for each column heading with each value separated by 2 spaces.

time    		Julian day, hours and minutes expressed as decimal

pres  		depth (m)

temp   		temperature degrees C

theta   		potential temperature, formulae from Fofonoff and Millard 1983

sal     		salinity ppt

sigma  		potential density, formulae from Fofonoff and Millard 1983

angle 		angle 0-90

flow   		flow counts 0000-9999

hzvel    		horizontal velocity (knots)

vtvel  		vertical velocity	(m/min)

vol   		seawater volume filtered  (m3)

net  		net number

fluor 		fluorescence (0-5 volts)

ptran    		extinction coefficient (0-5 volts)

oxycurrent 	0-5 volts

oxytemp   	0-5 volts internal to the probe

oxygen   	dissolved oxygen (ml/l)

lat 		latitude decimal degrees

lon		longitude decimal degrees

#### Raw filename *.raw

This file contains the raw data from the underwater unit. These “*.raw” files can be used in the playback mode to re-process the data (see page 33 MOCNESS instructions), and also serve as the backup in case there are problems with the processed data file.

For each tow, data is written (as it appears in the acquisition window) in ASCII to disk in a simple configuration, which consists of:

Rows 1-5		header information about sensors and the tow

Succeeding rows	raw data string formatted thusly: #MN- N1 N2 AA FFFF PPPPP PTPT TTTTTT CCCCCC BBB $GPGLL The fields are as follows and are discussed individually below.

- N1		net count, counts of left response switch, 00-99

- N2		net count, counts of right response switch, 00-99

- AA		net angle in degrees, 00-99

- FFFF	flow counts, 0000-9999. Reset to 0000 with every net response or increment net # button command

- PPPPP	pressure value, converted to decibars by the deck computer

- PTPT	a decimal number derived from the temperature sensor in the pressure sensor which is used to improve the pressure calibration

- TTTTTT	averaged SeaBird temperature period. The frequency from the SeaBird temperature sensor is measured and processed in the NCU to generate the value TTTTTT, a 6-digit decimal number. This number is scaled as follows:

(frequency of the temp. sensor)=K/(TTTTTT)

where K=1,258,291,200. Software in the deck computer uses this frequency value along with the individual sensor’s calibration file to calculate temperature.

- CCCCCC	averaged SeaBird conductivity period, handled exactly like the temperature frequency and the same scaling factor should be used

- BBB	battery voltage (divide value by 10)

- $GPGLL	latitude and longitude in decimal degrees

#### Tab filename *.tab

The statistical summary for a given *.pro file. For each net, the following parameters are included:

pmin,pmax,pavg 		minimum,maximum and average depth of net

tmin,tmax,tavg		minimum,maximum and average temperature of net

smin,smax,savg		minimum,maximum and average salinity of net

amin,amax,aavg		minimum,maximum and average angle of net

spmin,spmax,spavg		minimum,maximum and average horizontal velocity (kt) of net

armin,armax,aravg		minimum,maximum and average vertical velocity (m/sec) of net

#obs				total number of observations while net was open

vol				total seawater volume filtered for net

#### Captured screens, filenames *.gif

This file is a GIF image of the acquisition software captured at any point during a MOCNESS “flight”. The files on this CD reflect the acquisition window at the end of each “flight”, immediately prior to ending acquisition.

## GUV

GUV/

All GUV files, both binary and ASCII, have been zipped and placed in zip archives called csvfiles.zip and logfiles.zip respectively.

The GUV files were converted to CSV (Comma Separated Value) format using the Biospherical Instruments’ PUVPROF program.

The columnar format of the CSV file is listed below:

| Field | PUV Data |  | Field | GUV Data |
| --- | --- | --- | --- | --- |
| 1 | Depth |  | 1 | Depth |
| 2 | 308 U |  | 2 | 306 U |
| 3 | 320 U |  | 3 | 320 U |
| 4 | Temp U |  | 4 | Temp U |
| 5 | MATFL |  | 5 | MATFL |
| 6 | 340 U |  | 6 | 340 U |
| 7 | 380 U |  | 7 | 380 U |
| 8 | PAR U |  | 8 | PAR U |
| 9 | Temp U |  | 9 | Temp S |
| 10 | 308 S |  | 10 | 300 S |
| 11 | Batt S |  | 11 | Batt S |
| 12 | 320 |  | 12 | 320 S |
| 13 | 340 |  | 13 | 340 S |
| 14 | 380 |  | 14 | 380 S |
| 15 | Gnd |  | 15 | Gnd S |
| 16 | PAR |  | 16 | PAR S |
| 17 | K308 |  |  |  |
| 18 | K320 |  |  |  |
| 19 | K340 |  |  |  |
| 20 | K380 |  |  |  |
| 21 | Kpar |  |  |  |
| 22 | Chl |  |  |  |
| 23 | Prod |  |  |  |

## Isobar Charts

#### Isobars/

Forecasts of mean sea level pressure from the National Center for Environmental Prediction’s Medium Range Forecast Model.  Updated every 12 hours.

## This Report

Report/

Copies of this report in MS Word formats.

## RVDAS

##### JGOF/

The JGOFS data set consists of a single file produced each day named jg<julian_day>.dat.gz where <julian_day> is the day the data was acquired. The “.gz” extension indicates that the individual files are compressed before archiving. The daily file consists of 22 separate columnar fields in text format, which are described below. The JGOFS data set is obtained primarily by applying calibrations to raw data and decimating to whole minute intervals.  However, several fields are derived measurements from more than a single raw input.  Note: Null, unused, or unknown fields are filled with 9’s in the JGOFS data.

Additionally, 3 separate QC plots are generated daily by the ET using the JGOFS data set.  These plots include TSG and Bathymetry data, meteorological data, and navigation data.  The files are called ocean<julian_day>.ps, met<julian_day>.ps, and nav<julian_day>.ps respectively.

| Field | Data | Units |
| --- | --- | --- |
| 01 | GMT date | dd/mm/yy |
| 02 | GMT time | hh:mm:ss |
| 03 | PCOD latitude (negative is South) | Ddd.dddd |
| 04 | PCOD longitude (negative is West) | Ddd.dddd |
| 05 | Ships speed | Knots |
| 06 | GPS HDOP | - |
| 07 | Gyro Heading | Degrees (azimuth) |
| 08 | Course over ground | Degrees (azimuth) |
| 09 | Mast PAR | Einsteins/meters2 sec |
| 10 | Sea surface temperature | C |
| 11 | Not used | - |
| 12 | Sea surface salinity | PSU |
| 13 | Sea depth (uncorrected, calc. sw sound vel. 1500 m/s) | meters |
| 14 | True wind speed (port windbird) | meters/sec |
| 15 | True wind direction (port windbird) | degrees (azimuth) |
| 16 | Ambient air temperature | C |
| 17 | Relative humidity | % |
| 18 | Barometric pressure | mBars |
| 19 | Sea surface fluorometry | volts (0-5 FSO) |
| 20 | Not used | - |
| 21 | PSP | W/m2 |
| 22 | PIR | W/m2 |

## RVDAS

RVDAS (Research Vessel Data Acquisition System) was developed at Lamont-Doherty Earth Observatory of Columbia University and has been used on the R/V Maurice Ewing for several years.  It was adapted for use on the Nathaniel B. Palmer and her sister ship, the R/V Laurence M. Gould.

Below you will find detailed information on the data included. Be sure to read the “Significant Acquisition Events” section below for important information about data acquisition during this cruise.

#### Meteorological and Light Data

| Measurement | File ID | Collect. Status | Rate | Instrument |
| --- | --- | --- | --- | --- |
| Air Temperature | lmet | continuous | 1 sec | R. M. young 41372VC |
| Relative Humidity | lmet | continuous | 1 sec | R. M. young 41372VC |
| Wind Speed/Direction | lmet | continuous | 1 sec | R. M. young 5106 |
| PIR (LW radiation) | lmet | continuous | 1 sec | Eppley PIR |
| PSP (SW radiation) | lmet | continuous | 1 sec | Eppley PSP |
| Photosynthetically- Available Radiation | lmet | continuous | 1 sec | BSI QSR-240 |
| Barometer | lmet | continuous | 1 sec | R. M. young 61201 |

#### Navigational Data

| Measurement | File ID | Collect. Status | Rate | Instrument |
| --- | --- | --- | --- | --- |
| Attitude GPS | lash | continuous | 1 sec | Ashtec ADU-2 |
| P-Code GPS | lpcd | continuous | 1 sec | Trimble 20636-00SM |
| Gyro | lgyr | continuous | 0.2 sec | Yokogawa Gyroscope |
| Trimble GPS | tgps | continuous | 1 sec | NT200 |

#### Geophysical Data

| Measurement | File ID | Collect. Status | Rate | Instrument |
| --- | --- | --- | --- | --- |
| Bathymetry | lknu | variable | Varies | Knudsen 320B/R |

#### Oceanographic Data

| Measurement | File ID | Collect. Status | Rate | Instrument |
| --- | --- | --- | --- | --- |
| Salinity | ltsg | continuous | 15 sec | SeaBird 21 |
| Sea S Temperature | ltsg | continuous | 15 sec | SeaBird 3-01/S |
| Fluorometry (analog) | ltsg | continuous | 15 sec | Turner 10-AU-005 |
| ADCP | ladc | continuous | 1 sec | RD Instruments |

### Data File Names and Structures

RVDAS data is divided into two broad categories, Underway and Navigation.  The groups are abbreviated “uw” and “nav”. Thus, these two tar files, lmguw.tar and lmgnav.tar exist under the top-level rvdas directory. The instruments are broken down as shown. Each data file is g-zipped to save space on the distribution.  Not all data types are collected everyday or on every cruise.

RVDAS data files are named following the convention: LMG[FileID].dDDD.

- The FileID is a 4-character code representing the system being logged, for example: lmet (for meteorology)

- DDD is the Julian day of the data collection

| Underway Data | File ID |  | Navigation Data | File ID |
| --- | --- | --- | --- | --- |
| Meteorolical | lmet |  | Gyro Compass | lgyr |
| Knudsen | lknu |  | P-CODE GPS | lpcd |
| Thermosalinograph | ltsg |  | Ashtech ADU2 GPS | lash |
| ADCP | ladc |  | Trimble NT2100 GPS | tgps |
| Sound Velocity Probe | lsvp |  |  |  |

Data is received by the RVDAS system via RS-232 serial connections. The data files that comprise the rvdas data set are described below. A time tag is added to each line of data received and the data is written to disk.

YY+DDD:HH:MM:SS.SSS [data stream from instrument]

Where, YY: two-digit year, DDD: Julian Day, HH: 2 digit hours, MM: 2 digit minutes SS.SSS: seconds.  All times are UTC.

The delimiters used to separate fields in the raw data files are usually spaces and commas, but other delimiters are used (::, =, @) and occasionally there is no delimiter.  Care should be taken when reprocessing the data that the fields separations are clearly understood.  An example data

#### lknu

99+099:00:18:19.775 hf,305.2,lf,304.3

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | hf – high frequency flag (12 kHz) |  |
| 3 | high fequency depth | meters |
| 4 | lf – low frequency flag (3.5 kHz) |  |
| 5 | low frequency depth | meters |

#### lmet

00+040:00:00:00.928 16-10-97 00:01:19 005.53 005.53 005.69 145 141 005.15 005.12 005.15

- 146 148 004.4 004.4 004.4 94.07 94.06 94.07 0981.40 -0.023 00.292 00.315

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | Instrument date | dd-mm-yy |
| 3 | Instrument time | hh:mm:ss |
| 4 | Port wind speed (10 second running average) | m/s |
| 5 | Port wind speed (min for 24 hour cycle) | m/s |
| 6 | Port wind speed (max for 24 hour cycle) | m/s |
| 7 | Port wind direction | degrees |
| 8 | Port wind direction (standard deviation) | degrees |
| 9 | Starboard wind speed (10 second running average) | m/s |
| 10 | Starboard wind speed (min for 24 hour cycle) | m/s |
| 11 | Starboard wind speed (max for 24 hour cycle) | m/s |
| 12 | Starboard wind direction | degrees |
| 13 | Starboard wind direction (standard deviation) | degrees |
| 14 | Temperature | C |
| 15 | Temperature (min for 24 hour cycle) | C |
| 16 | Temperature (max for 24 hour cycle) | C |
| 17 | Relative humidity | % |
| 18 | Relative humidity (min for 24 hour cycle) | % |
| 19 | Relative humidity (max for 24 hour cycle) | % |
| 20 | Barometric pressure | millibars |
| 21 | PSP (long wave radiometer) | Volts |
| 22 | PIR (short wave radiometer) | Volts |
| 23 | PAR (photo-synthetically available radiation, 400 - 700 nm) | Volts |

#### svp1

00+348:01:59:52.128  1539.40

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | Sound velocity | m/s |

#### ltsg

00+040:00:00:06.153 6926 15.8827 0.2010 2.9218 3.239 4.334e-01

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | Scan number |  |
| 3 | Internal water temperature | C |
| 4 | Salinity | PSU |
| 5 | External water temperature | C |
| 6 | Transmissometer signal | Volts |
| 7 | Fluorometer signal (analog) | Volts |

#### ladc

00+019:23:59:59.099 $PUHAW,UVH,-1.48,-0.51,250.6

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | $PUHAW |  |
| 3 | UVH (E-W, N-S, Heading) |  |
| 4 | Ship Speed relative to reference layer1 velocity2, East vector | knots |
| 5 | Ship Speed relative to reference layer1 velocity2, North vector | knots |
| 6 | Ship heading | degrees |

1The reference layer is an average velocity measured in a number of depth “bins”. On the LMG, the bins are eight meters deep and bins 3-10 define the reference layer. Hence, the reference layer is the water column from 16-80 meters beneath the ship.

2The speed ouput is water velocity relative to the ship’s hull and is therefore opposite of the actual

movement of the ship. For example, if the ship’s heading is due north, the North/South reference layer

velocity is likely to be negative (southerly).

#### lash

##### ATTD: Attitude Data

01+081:00:00:00.806 $PASHR,ATT,345605.0,165.03,+001.86,-01.96,0.0018,0.0173,0*22

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $PASHR |  |
| 2 | ATT |  |
| 3 | GPS Time sec. of the week | seconds |
| 4 | heading (rel. to true North) | degrees |
| 5 | pitch | degrees |
| 6 | roll | degrees |
| 7 | Measuremnet RMS error | meters |
| 8 | Baseline RMS error | meters |
| 9 | attitude reset flag |  |

##### GGA: GPS Position Fix – Geoid

01+081:00:00:00.966 $GPGGA,235952.00,6051.7937,S,06030.2175,W,1,08,01.0,+00068,M,,M,,*79

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPGGA |  |
| 2 | $GPGGA, NEMA sentence label |  |
| 3 | UTC time at position | hhmmss.ss |
| 4 | Latitude | ddmm.mmm |
| 5 | North (N) or South (S) |  |
| 6 | Longitude | ddmm.mmm |
| 7 | East (E) or West (W) |  |
| 8 | GPS quality (1=GPS 2=DGPS) |  |
| 9 | Number of GPS satellites used |  |
| 10 | HDOP |  |
| 11 | Antenna Height | meters |
| 12 | M for Meters |  |
| 13 | Geoidal height | meters |
| 14 | M for meters |  |
| 15 | age of diff. GPS data | seconds |
| 16 | differential reference station ID |  |

#### lgyr

01+155:00:00:00.106 $IIVHW,001.2,T,,M,,N,,K*78

01+155:00:00:00.286 $HEHDT,001.2,T*2C

01+155:00:00:00.486 $HEROT,004.3,A*2C

01+155:00:00:00.665 $HCHDT,,T*07

01+155:00:00:00.885 $PASVW,06.0,A*1A

##### HDT: True Heading

01+083:00:00:02.893 $HEHDT,246.3,T*2C

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $HEHDT |  |
| 2 | Heading XXXXX = ddd.d | degrees |
| 3 | T flag for true heading, checksum |  |

##### ROT: Rate of Turn

01+083:00:00:03.093 $HEROT,-006.3,A*03

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $HEROT |  |
| 2 | Rate of turn | degrees/min |
| 3 | Status: A = data valid, checksum |  |

#### tgps

##### GGA: Global Positioning Fix Data

00+040:00:00:00.985 $GPGGA,000003,6139.961,S,05949.422,W,1,6,001.64,-00036,M,00000,M,,

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPGGA |  |
| 2 | Latitude in degrees with decimal minutes | ddmm.mmm |
| 3 | North (N) or South (S) |  |
| 4 | Longitude in degrees with decimal minutes | ddmm.mmm |
| 5 | East (E) or West (W) |  |
| 6 | GPS quality (1=GPS 2=DGPS) |  |
| 7 | Number of GPS satellites used |  |
| 8 | Horizontal dilution of precision (HDOP) |  |
| 9 | Antenna height above/below mean-sea-level (geoid) | meters |
| 10 | Units for antenna height (M = Meters) |  |
| 11 | Geoidal Separation1 |  |
| 12 | Units for Geoidal Separation (M = Meters) | meters |
| 13 | Age of differential GPS data, number of seconds since last SC104 Type 1 or 9 |  |
| 14 | Differental reference station ID |  |

1Geoidal Separation: the difference between the WGS-84 earth ellipsoid and mean-sea-level (geoid). A negative value represents mean-sea-level below ellipsoid.

##### GLL: Geographic Position – Latitiude/Logitude

00+040:00:00:00.065 $GPGLL,6139.96,S,05949.42,W,000002,A

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPGLL |  |
| 2 | Latitude | ddmm.mmm |
| 3 | North (N) or South (S) |  |
| 4 | Logitude | ddmm.mmm |
| 5 | East (E) or West (W) |  |
| 6 | UTC of position | hhmmss.ss |
| 7 | Status: A = Data Valid |  |

##### VTG: Track Made Good and Speed over Ground

00+040:00:00:00.213 $GPVTG,161,T,149,M,009.6,N,017.8,K

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPVTG |  |
| 2 | Track, degrees true | degrees |
| 3 | T flag for True |  |
| 4 | Track, degrees magnetic | degrees |
| 5 | M flag for Magnetic |  |
| 6 | Speed over Ground | knots |
| 7 | N flag for Knots |  |
| 8 | Speed over Ground | kmhr |
| 9 | K flag for km/hr |  |

##### VHW: Speed Through Water and Heading

00+040:00:00:00.212 $GPVHW,246,T,234,M,012.3,N,022.8,K

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPVHW |  |
| 2 | Heading, degrees True | degrees |
| 3 | T flag for True |  |
| 4 | Heading, degrees Magnetic | degrees |
| 5 | M flag for Magnetic |  |
| 6 | Speed through water | knots |
| 7 | N flag for Knots |  |
| 8 | Speed through water | km/hr |
| 9 | K flag for km/hr |  |

##### ZDA: Time and Date

00+040:00:00:00.285 $GPZDA,000002,09,02,2000,00,00

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPZDA |  |
| 2 | UTC time | hhmmss.ss |
| 3 | Day: 01 – 31 | dd |
| 4 | Month: 01 – 12 | mm |
| 5 | Year | yyy |
| 6 | Local time zone description1, 00 +/-13 hrs |  |
| 7 | Local time zone minutes description, same sign as local hours |  |

1Zone description is the number of whole hours added to local time to obtain GMT, values are negative for East longitudes.

##### BWC: Bearing and Distance to Waypoint

00+040:00:00:00.865 $GPBWC,000003,6209.70,S,05824.00,W,127.2,T,115.3,M,050.1,N,014

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPBWC |  |
| 2 | UTC of bearing | hhmmss.ss |
| 3 | Destination waypoint latitude in degrees, decimal minutes | ddmm.mmm |
| 4 | Hemisphere Flag: N or S |  |
| 5 | Destination waypoint longitude in degrees, decimal minutes | ddmm.mmm |
| 6 | Hemisphere Flag: E or W |  |
| 7 | Bearing, degrees true | degrees |
| 8 | T flag for True |  |
| 9 | Bearing, degrees magnetic | degrees |
| 10 | M flag for Magnetic |  |
| 11 | Distance to waypoint in nautical miles | nm |
| 12 | N flag for Nautical Miles |  |
| 13 | Waypoint ID |  |

#### lpcd

##### GGA: GPS Position Fix – Geoid/Ellipsoid

00+019:23:59:59.301 $GPGGA,235958.409,6849.6944,S,13712.8472,W,1,06,1.2,092.4,M,047.3,M,,*67

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPGGA |  |
| 2 | UTC time at position | hhmmss.sss |
| 3 | Latitude | ddmm.mmm |
| 4 | North (N) or South (S) |  |
| 5 | Longitude | ddmm.mmm |
| 6 | East (E) or West (W) |  |
| 7 | GPS quality (1=GPS 2=DGPS 3=P-CODE) |  |
| 8 | Number of GPS satellites used |  |
| 9 | HDOP |  |
| 10 | Antenna Height | meters |
| 11 | M for Meters |  |
| 12 | Geoidal height | meters |
| 13 | M for meters |  |
| 14 | Null field |  |
| 15 | Checksum |  |

##### GLL: GPS Latitude/Longitude

00+019:23:59:59.381 $GPGLL,6849.6944,S,13712.8472,W,235958.409,A*35

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPGLL |  |
| 2 | Latitude | degrees |
| 3 | North or South |  |
| 4 | Longitude | degrees |
| 5 | East or West |  |
| 6 | UTC of position | hhmmss.sss |
| 7 | staus of data (A = valid) |  |
| 8 | Checksum |  |

##### VTG: GPS Track and Ground Speed

00+019:23:59:59.382 $GPVTG,238.7,T,182.3,M,001.8,N,003.3,K*41

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPVTG |  |
| 2 | Heading | degrees |
| 3 | degrees True (T) |  |
| 4 | Heading | degrees |
| 5 | degrees magnetic (M) |  |
| 6 | Ship speed | knots |
| 7 | N = knots |  |
| 8 | Speed | km/hr |
| 9 | Checksum |  |

## Processing Notes

Refer to the instrument.coeff file, included in the root level of the data distribution, along with the specific instrument calibration sheets, found at the end of this report, for information on how the RVDAS data was processed.

## XBT

XBT/

During the cruise Expendable Bathythermographs were used to obtain water column temperature profiles.  The data from 14 casts are included. ASCII-readable files are contained in EDF.zip, while the raw data files produced by the MK12 software are provided in RDF.zip. XBTPlots contains graphics produced from the *.EDF files using Microsoft Excel.

# LMG01_06 Sensors

The most recent calibration sheets for CTD Instrumentation, Mast Radiometer Instrumenation, and TSG Instrumenation have been included on the CD in the Cal/ directory.  These are scanned image files of the hard copy calibration sheets.  A key to match filenames to instruments is shown below.

TSG Primary Temperature (Internal) s/n 1577

TSG Primary Temperature - after 1577 failure - s/n 1789

TSG Secondary temperature (Sea Water Intake) s/n 1619

TSG Conductivity s/n 1577

TSG Conductivity - after 1577 failure - s/n 1789

Underway Transmissometer s/n 168

CTD Primary Temperature s/n 2470

CTD Secondary Temperature s/n 1529

CTD Primary Conductivity s/n 2065

CTD Secondary Conductivity s/n 2247

CTD Primary Pump s/n 823

CTD Secondary Pump s/n 995

CTD PAR s/n 4472

CTD Transmissometer s/n 407

CTD Dissolved Oxygen s/n 545

CTD Fluorometer s/n 093

CTD Fish s/n `91480

Mast PAR s/n 6394

Mast PSP s/n 31701F3

Mast PIR s/n s/n 28903F3

MOC1 Depth- initial and last two tows - s/n 146

MOC1 Depth- after change - s/n 139

Port Anemometer s/n 35061

Starboard Anemometer s/n 28394

Barometer s/n BP01150 - no cal available

Humidity/Wet Temperature s/n 4018

GUV s/n 9266

## Additonal information about Shipboard Sensors

| Sensor | Description | Serial # | Cal. Date | Status |
| --- | --- | --- | --- | --- |
| Port Anemometer | R.M. Young 105106 | VM35061 | 04/15/01 | collect |
| Stbd Anemometer | R.M. Young 105106 | VM28394 | 04/15/01 | collect |
| Barometer | R.M. Young 61201 | BP01150 | 11/07/00 | collect |
| Humidity/Wet Temp | R.M. Young 41372VC | 4018 | 11/07/01 | collect |
| Mast PAR | BSI QSR-240 | 6394 | 06/05/01 | collect |
| Pyranometer | Eppley PSP | 31701F3 | 06/01/02 | collect |
| Pyrgeometer | Eppley PIR | 32031F3 | 06/01/01 | collect |
| GUV | Biospherical GUV-511 | 9266 | 11/08/00 |  |
| TSG | SeaBird SBE21 | 219209-1577 | 05/30/01 | initial |
| TSG | SeaBird SBE21 | 2110410-1789 | 02/11/00 | collect |
| TSG Remote Temp | SeaBird 3-01/S | 1619 | 06/02/01 | collect |
| Fluorometer | Turner 10-AU-005 Lamp: daylight 10-045, reference filter: 10-052, emission filter: 10-051, excitation filter: 10-050. | 6046RTD |  | collect |
| Transmissometer | WET Labs 9707017 | CST-168R | 11/30/00 | collect |
| P-Code GPS | Trimble 20636-00 (SM) | 220035265 | 04/01/01 | PCD/CIV |
| Bathymetry | Knudsen 320B/R | K97715 |  |  |

## LMG01_06 CTD Sensors:

| Sensor | Description | Serial # | Cal. Date | Status |
| --- | --- | --- | --- | --- |
| CTD Fish | Sea-Bird 9Plus-3400m | 91480 | 06/23/01 |  |
| CTD Deck Unit | Sea-Bird 11Plus | 111411 |  |  |
| CTD Deck Unit- spare | Sea-Bird 11Plus | 11P8908-0288 |  |  |
| Prim. Temp. Sensor | Sea-Bird 3-02/F | 2470 | 1/11/01 |  |
| Sec. Temp. Sensor | Sea-Bird 3-02/F | 1529 | 08/22/00 |  |
| Prim. Cond. Sensor | Sea-Bird 4-02/0 | 2065 | 02/21/01 |  |
| Sec. Cond. Sensor | Sea-Bird 4C | 2247 | 02/21/01 |  |
| Diss. Oxygen Sensor | Sea-Bird 13-02-B | 130545 | 02/22/00 |  |
| Fluorometer | Chelsea model Mk III Aquatracka | 2050/093 | 05/04/00 |  |
| Transmissometer | Wet-Labs C-Star 25cm | CST-407DR | 11/16/00 |  |
| PAR | Biospherical QSP-200L4S | 4472 | 07/06/99 |  |

# Acquisition Problems and Events

Briefly, the three problems of note were: 1) failure of first TSG, apparently due to fouling. It was installed dirty, after it had been calibrated but not refurbished;. 2)failure of ADCP raw data collection August 16 --no raw data files were recorded after that date; 3) general problems with MOCNESS pressure sensors; and 4) possible error in met data due to unknown amplification of raw sensor signals --particularly PSP.

This section lists all known problems with acquisition during this cruise including instrument failures, data acquisition system failures and any other factor affecting this data set. The format is jjj:hhmm (jjj is julian day, hh is hour, and mm is minute).  All times are in GMT.

202	2200	Start DAS, start Knudson (note that data for j.d. 202 through 204 was deleted from

record because we were within the 200-mile limit of Argentine waters.

204	1946	Cyclone (DAS computer) crash

204	2005	Cyclone back up.

206	1732	Stopped GUV logging to check that it was logging. Restarted within minutes.

206	2049	Palmer Station - turn off TSG / pump.

206	2053	Stopped ADCP

207	1945	Had to start ADCP about 5 times before it stayed on. Used old start.cnf file.

207	2040	Restarted Knudson sounder.

207	2042	Resume logging Knudson & ADCP.

207	2045	Restarted logging on Gentoo.

207	2050	Restarted TSG logging.

207	2350	Stopped ADCP to edit start.cnf file.

208	0000	Restarted ADCP OK --manual edit of start.cnf file worked.

208	1047	XBT attempt --abort in ice.

208	1051	XBT attempt --abort in ice

208	1054	XBT OK to ~260m; depth 600m @ S65*22' W066* 28'

208	1057	XBT good all.

208	1210	GUV stopped for file transfer

208	1221	GUV resumed logging

208	1233	Restarted file P07271AB on GUV.

208	1957	TSG salinity low, icing.

209	0050	GUV data interruption for file transfer

209	2249	Discovered that Felix "Sat" back-up failed

210	0845	GUV data interruption for file transfer.

210	1648	TSG readings bad. Cleaned, restarted; sal. Now 14 instead of 24!

210	1721	Stop  ADCP for data back-up.

210	1735	Restart ADCP.

210	2120	First MOCNESS tow --problems with depth sensor.

211	0055	TSG logging still off because data bad…

211	2030	Installed TSG S/N 1789. Data looks good.

211	0842	Noticed TSG xmisso. Voltage now more believable at 4.443V --was it iced?

213	1417	MOCNESS attempts --pressure sensor frozen. Replaced diaphragm.

213	2210	Stopped  ADCP (on station) for data back-up.

213	2220	Restarted ADCP.

215	0211	MOCNESS tow --pressure sensor goofy. Changed from NCU 146 to NVU 139.

Overhauled salt-contaminated tube on #146.

216	0220	Problem with MOCNESS net release response.

219	1400	CTD conductivity sensor problems. Causre probably ice. BUT, discover problems with

surface PAR input. Discovered it had been wired wrong.

219	2300	Rad van heating problems addressed.

220	1723	Investigated PSP --indicated 2009 W/m2 too high, but cal coeff. Look OK.

221	1930	Stopped ADCP for data back-up.

222	2320	GUV interrupted for file transfer.

223	2355	GUV interrupted for file transfer.

226	0043	GUV interrupted for file transfer.

229	0019	GUV interrupted for file transfer

230	0004	GUV interrupted for file transfer.

230	0015	Stopped ADCP for data back-up.

230	0057	Restarted ADCP --on station.

231	1000	Network server crash.

231	1120	Cyclone (DAS computer) message: "Memory parity error in SIMM S4."

231	1205	Network server back up.

234	2357	GUV interrupted for file transfer.

235	1135	Felix did not eject tape after routine back-up.

235	1720	Found TSG bottom drain valve open! Closed it.

237	0856	Felix "Sat" back-up failed. 2nd try OK.

237	1430	MOCNESS NCU 139 failed when doused with salt water. NCU 146 (overhauled)

put back on.

237	1445	MOCNESS struck ice, damaging temp. sensor and flow meter. After tow, sensor

S/N 2526 (slow response, damaged) replaced by S/N 1577 (fast response).

237	1830	Discovered error in MOCNESS temp. sensor S/N 1577 cal. Coeff. Entered. Corrected.

238	1406	At Palmer Station, stop TSG logging.

238	1412	Stop ADCP logging.

239	0130	Some trouble with QFAX keyboard.

239	1300	Discovered ADCP had stopped logging raw data August 16. 746 files on E:. Disk not full.

239	1504	Started ADCP with old start.cnf file from LMG0105.

239	2230	Stopped ADCP to check number of files on E:.

239	2250	Restart ADCP with old start.cnf file. No DAS logging! Later realize it doesn'e have com 2

set to transmit.

239	2318	Turned ADCP bottom-tracking off (had been on).

240	0120	GUV interrupted for file transfer

240	1008	Stopped ADCP to look at com port config. Restarted within minutes.

240	1045	Mounted felix:/public on cyclone:/public.

240	1205	ADCP stop in Deception Island to try other ue4.cnf. Set com 2 to transmit.

240	1217	ADCP restart OK.

240	2240	Trimble GPS has trouble tracking --suspect antenna connection. (NOT P-code.)

241	1320	Found NERA antenna pedestal mount bolts loose.

241	2040	At 200-mile limit, stopped DAS for preparation of cruise CD.

242	1457	Discover that I cannot even delete files from ADCP E: drive!