Progress Report.

September 1, 2002

LMG0205

Nutrients and O2

Two Rosette casts have been deployed, sampled and analyzed.  The data for all casts has been placed in event files and copied to the science drive.  In addition, nutrient and oxygen analysis associated with the hand Niskin deployment has been completed and placed in a surface file on the science drive.  Concern has arisen about the validity of the data generated by the SBE oxygen sensor (CTD).  All titration data has been compiled with oxygen concentrations measured by the CTD sensors.  Comparison of the two procedures shows considerable variation.  Oxygen concentrations generated by the SBE sensor also shows considerable variation when compared to other state properties generated by the CTD package (Salinity, Temperature). The exact reason for the disparity has not been discovered but is being investigated.

Data associated with Dr. Fritsens ice samples has gone through post processing and has been delivered.  Initial analysis of this data shows a strong relationship with salinity, indicating fidelity in the analytical procedure.