- LMG0302 - GLOBEC

- Cruise Data Report

By Todd B. Johnson and Andy Nunn

February 14, 2003 – March 7, 2003

Table of Contents

Introduction	1

Archive Data Extraction	2

CD Directory Structure	3

Distribution Contents	4

ADCP	4

Calibration	4

CTD	4

/CTD/raw	4

/CTD/process	4

Cruise Track	5

Ice Images	5

Isobar Charts	5

Data and Science Report	5

QC Plots	5

XBT	5

/XBT/	5

Science and Mooring Data	5

JGOFS Data Set	6

RVDAS	6

Meteorological and Light Data	6

Navigational Data	7

Geophysical Data	7

Oceanographic Data	7

Data File Names and Structures	8

lknu	8

lmet	8

ltsg	9

lpco	9

svp1	9

ladc	9

lash	10

lgyr	10

tgps	11

lpcd	13

LMG Sensors	15

Shipboard Sensors	15

CTD Sensors	15

Acquisition and Processing Information	16

Processing Specifics	16

Errors and Events	16

# Introduction

The LMG data acquisition systems continuously log data from a suite of instrumentation throughout the cruise.  This document describes the format of that data and its location on the distribution CDs.  It also contains important information which may affect how this data is processed such as instrument failures or other known problems with acquisition.

The data collected during this cruise is distributed on a CD-ROM written in ISO9660 level-1 format.  This data format has very strict requirements on filenames and organization.  However, it is readable by virtually every computing platform.

All of the data has been archived with the Unix “tar” command and/or compressed using Unix “gzip” compression.  Tar files have a “.tar” extension and Gzipped files have a “.gz” extension.  Tools are available on all platforms for uncompressing and de-archiving these formats.  On Macintosh, Stuffit Expander with DropStuff will open a tar archive and uncompress gzipped and Unix compressed files.  For Windows9X, WinZip, a shareware utility included on this CD (remember, it is shareware) will open these files.

In some cases to adhere to the ISO9660 format the .tar extension was removed.  When we tarred the files then gzip the tar archive the name of the file became File.tar.gz.  This name does not follow the 8.3 naming convention of the ISO9660 format.  So the file was renamed without .tar making the file name File.gz.  On Windows and Mac Platforms Winzip and Stuffit Expander handles this just fine.  When they expand the File.gz the expanded file becomes File.tar, which both software packages can handle.  On Unix platform gunzip expands File.gz but it does not append the .tar extension.  So you may not recognize the file as a tar archive, but OS does recognize it as a tar archive.  If you use the file command it will return saying it is a tar file.  The below tar command will un-archive the file just fine.

IMPORTANT:  Read the last section in this document, Acquisition Problems and Events, for important information that may affect the processing of this data.

# Archive Data Extraction

It is often useful to know exactly how an archive was produced when expanding its contents.  Tar files were created using the following commands:

tar cvf archive-file files-to-be-archived

To create a list of the files in the archive:

tar tvf  archive-file  > contents.list

To extract the files from the archive:

tar xvf  archive-file file(s)-to-extract

G-zipped files will have a “.gz” extension on the filename. These files can be decompressed after de-archiving, using:

gunzip filename.gz

# CD Directory Structure

| /ADCP/ Pingdata/ Gentoo/ #### /CAL/ SVP_CALS.tar UW_CALS.tar CTD_CALS.zip MET_CALS.zip Instrument.coeff #### /CTD/ Raw/ Process/ #### /ICE_IMAGE/ *.jpg /ISOBARS/ *.gif /JGOF/ lmgjgof.tar /MAPS/ *.jpg *.ps Track.txt /MOORING/ Moor2003.zip | /QC_PLOTS/ Lmgqc.tar /REPORT/ Report.doc Report.html Report.txt /RVDAS/ Lmguw.tar Lmgnav.tar /SCIENCE/ Report/ Met_Data/ /SITREPS/ *.txt /UTILITY/ Winzip Stuffit Expander /XBT/ *.EDF *.RDF |
| --- | --- |

# Distribution Contents

## ADCP

- /Adcp/

The ADCP DAS data files are named pingdata.xxx (xxx representing a file number).  Note that these extensions do NOT represent Julian day numbers. Please refer to the file’s creation date.  The ADCP DAS computer creates a new pingdata file when the current one reaches a size of 320K.  The ping files logged on Gentoo, the Linux processing computer, however are created new each day.

Some ADCP data is also transmitted to RVDAS.  East and North vectors for ship’s speed relative to the reference layer and ship’s heading are archived in the navigational data section of RVDAS.

## Calibration

/Cal/

The tar files in the Cal directory contain images of calibration sheets for each of the following systems:  Sound Velocity Probe(SVP_CALS.TAR), Meteorological System(MET_CALS.TAR), Underway System(UW_CALS.TAR), and CTD(CTD_CALS.TAR).

CTD diagrams can be found in the ctd Cal directory for voltage and frequency sensors.

## CTD

/CTD/

The ctd data was collected and processed on a windows 98 computer, using Seasave Win32 – Version 5.25a and SBE Data Processing – Version 5.21.

For more information and software visit the web site at www.seabird.com.

### /CTD/raw

This directory contains the original .con, .dat, .bl, and .hdr files files produced for each CTD cast.

### /CTD/process

Each processed cast is represented here by a set of ten files. Note that cast 1 was split into a separate data set for the up and down segments. Also, the DO2 sensor was only added onto to the instrument package for the 4th cast. In addition to the standard files listed below, an additional round of post processing was done to produce rg302EEE.cnv and .asc files containing non-bin averaged data and an ascii conversion of that data.

| g302EEE.con | A copy of the configuration file for the cast. |
| --- | --- |
| g302EEE.cnv | The converted file for the whole cast. |
| g302EEE.ros | The rosette file that contains the scan lines for each bottle trip. |
| g302EEE.btl | The bottle file that contains the avg, standard deviation, min, and max for a select set of variables for each bottled fired during the upcast. |
| dg302EEE.cnv | The converted file for the down cast. |
| dg302EEE.asc | An ASCII formatted file for the down cast without a header. |
| dg302EEE.hdr | The header for the down cast. |
| ug302EEE.cnv | The converted file for the up cast. |
| ug302EEE.asc | An ASCII formatted file for the up cast without a header. |
| ug302EEE.hdr | The header for the up cast. |

## Cruise Track

/MAP/

PostScript cruise track plots have been produced for this cruise, in three sizes. It has also been converted to a jpeg format.  The cruise track file track.txt is also included and contains the latitude and longitude in 10-minute intervals.

## Ice Images

/ICE_IMAGE/

This directory contains .jpg files of Terrascan ice imagery sent to the ship from Palmer station before and during the cruise.

## Isobar Charts

- /Isobars/

This directory contains GIF image files.  These file are an analysis of mean sea level pressure from the National Center for Environmental Prediction's Medium Range Forecast Model.  They are updated every 6 hours.  Naming the convention is as follows yyjjj.hh.gif  where yy is the year, jjj is the day number, and hh is the hour.

## Data and Science Report

- /Report/

Copies of this report in MS Word, HTML, and text formats.

## QC Plots

/QC_PLOTS/

Postscript files of data stored each day on RVDAS for quality control analysis during the cruise.  There are 3 types of files, named metXXX.ps, navXXX.ps, and oceanXXX.ps, where XXX is represents the Julian day.  Met files are a summary of the data from the meteorological instruments, Nav files are a summary of navigational data, and Ocean files are a summary of the underway seawater and bathymetry data.

## XBT

### /XBT/

Expendable Bathythermographic (XBT) probes were used to obtain water column temperature profiles.  These XBT were launched at all SIO Mooring sites. The .RDF files contain the raw data, and the .EDF contained the exported ascii data.

# Science and Mooring Data

/Science/ and /Moor2003/

These directories contain additional data provided by the PI to supplement the standard distribution data set.

## JGOFS Data Set

- /JGOF/

The JGOFS data set consists of a single file produced each day named jg<julian_day>.dat.gz where <julian_day> is the day the data was acquired. The “.gz” extension indicates that the individual files are compressed before archiving. The daily file consists of 22 separate columnar fields in text format, which are described below. The JGOFS data set is obtained primarily by applying calibrations to raw data and decimating to whole minute intervals.  However, several fields are derived measurements from more than a single raw input.  Note: Null, unused, or unknown fields are filled with 9’s in the JGOFS data.

Additionally, 3 separate QC plots are generated daily by the ET using the JGOFS data set.  These plots include TSG and Bathymetry data, meteorological data, and navigation data.  The files are called ocean<julian_day>.ps, met<julian_day>.ps, and nav<julian_day>.ps respectively.

| Field | Data | Units |
| --- | --- | --- |
| 01 | GMT date | dd/mm/yy |
| 02 | GMT time | hh:mm:ss |
| 03 | PCOD latitude (negative is South) | Ddd.dddd |
| 04 | PCOD longitude (negative is West) | Ddd.dddd |
| 05 | Ships speed | Knots |
| 06 | GPS HDOP | - |
| 07 | Gyro Heading | Degrees (azimuth) |
| 08 | Course over ground | Degrees (azimuth) |
| 09 | Mast PAR | Einsteins/meters2 sec |
| 10 | Sea surface temperature | C |
| 11 | Not used | - |
| 12 | Sea surface salinity | PSU |
| 13 | Sea depth (uncorrected, calc. sw sound vel. 1500 m/s) | meters |
| 14 | True wind speed (port windbird) | meters/sec |
| 15 | True wind direction (port windbird) | degrees (azimuth) |
| 16 | Ambient air temperature | C |
| 17 | Relative humidity | % |
| 18 | Barometric pressure | mBars |
| 19 | Sea surface fluorometry | volts (0-5 FSO) |
| 20 | Not used | - |
| 21 | PSP | W/m2 |
| 22 | PIR | W/m2 |

## RVDAS

/rvdas/

RVDAS (Research Vessel Data Acquisition System) was developed at Lamont-Doherty Earth Observatory of Columbia University and has been used on the R/V Maurice Ewing for several years.  It was adapted for use on the Nathaniel B. Palmer and her sister ship, the R/V Laurence M. Gould.

Below you will find detailed information on the data included. Be sure to read the “Significant Acquisition Events” section below for important information about data acquisition during this cruise.

### Meteorological and Light Data

| Measurement | File ID | Collect. Status | Rate | Instrument |
| --- | --- | --- | --- | --- |
| Air Temperature | lmet | continuous | 1 sec | R. M. young 41372VC |
| Relative Humidity | lmet | continuous | 1 sec | R. M. young 41372VC |
| Wind Speed/Direction | lmet | continuous | 1 sec | R. M. young 5106 |
| PIR (LW radiation) | lmet | continuous | 1 sec | Eppley PIR |
| PSP (SW radiation) | lmet | continuous | 1 sec | Eppley PSP |
| Photosynthetically- Available Radiation | lmet | continuous | 1 sec | BSI QSR-240 |
| Barometer | lmet | continuous | 1 sec | R. M. young 61201 |

### Navigational Data

| Measurement | File ID | Collect. Status | Rate | Instrument |
| --- | --- | --- | --- | --- |
| Attitude GPS | lash | continuous | 1 sec | Ashtec ADU-2 |
| P-Code GPS | lpcd | Not collected | 1 sec | Trimble 20636-00SM |
| Gyro | lgyr | continuous | 0.2 sec | Anschutz Gyro |
| Trimble GPS | tgps | continuous | 1 sec | NT200 |

### Geophysical Data

| Measurement | File ID | Collect. Status | Rate | Instrument |
| --- | --- | --- | --- | --- |
| Bathymetry | lknu | variable | Varies | Knudsen 320B/R |

### Oceanographic Data

| Measurement | File ID | Collect. Status | Rate | Instrument |
| --- | --- | --- | --- | --- |
| Salinity | ltsg | continuous | 15 sec | SeaBird 21 |
| Sea S Temperature | ltsg | continuous | 15 sec | SeaBird 3-01/S |
| Fluorometry (analog) | ltsg | continuous | 15 sec | Turner 10-AU-005 |
| ADCP | ladc | continuous | 1 sec | RD Instruments |

# Data File Names and Structures

RVDAS data is divided into two broad categories, Underway and Navigation.  The groups are abbreviated “uw” and “nav”. Thus, these two tar files, lmguw.tar and lmgnav.tar exist under the top-level rvdas directory. The instruments are broken down as shown. Each data file is g-zipped to save space on the distribution.  Not all data types are collected everyday or on every cruise.

RVDAS data files are named following the convention: LMG[FileID].dDDD.

- The FileID is a 4-character code representing the system being logged, for example: lmet (for meteorology)

- DDD is the Julian day of the data collection

| Underway Data | File ID |  | Navigation Data | File ID |
| --- | --- | --- | --- | --- |
| Meteorolical | lmet |  | Gyro Compass | lgyr |
| Knudsen | lknu |  | P-CODE GPS | lpcd |
| Thermosalinograph | ltsg |  | Ashtech ADU2 GPS | lash |
| ADCP | ladc |  | Trimble NT2100 GPS | tgps |
| Sound Velocity Probe | lsvp |  | PCO2 System | lpco |

Data is received by the RVDAS system via RS-232 serial connections. The data files that comprise the rvdas data set are described below. A time tag is added to each line of data received and the data is written to disk.

YY+DDD:HH:MM:SS.SSS [data stream from instrument]

Where, YY: two-digit year, DDD: Julian Day, HH: 2 digit hours, MM: 2 digit minutes SS.SSS: seconds.  All times are UTC.

The delimiters used to separate fields in the raw data files are usually spaces and commas, but other delimiters are used (::, =, @) and occasionally there is no delimiter.  Care should be taken when reprocessing the data that the fields separations are clearly understood.  An example data

## lknu

99+099:00:18:19.775 hf,305.2,lf,304.3

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | hf – high frequency flag (12 kHz) |  |
| 3 | high fequency depth | meters |
| 4 | lf – low frequency flag (3.5 kHz) |  |
| 5 | low frequency depth | meters |

## lmet

- 02+314:23:59:50.067 01.2 047 028 01.3 063 042 0988.8 001.7 084 -000.2192 0000.9358 0025.5875

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | Port Wind Speed | m/s |
| 3 | Port Wind Direction | deg |
| 4 | Port Wind Direction (standard deviation) | deg |
| 5 | Starboard Wind Speed | m/s |
| 6 | Starboard Wind Direction | deg |
| 7 | Starboard Wind Direction (standard deviation) | deg |
| 8 | Barometer | millibars |
| 9 | Temperature | C |
| 10 | Relative humidity | % |
| 11 | PSP (long wave radiometer) | Volts |
| 12 | PIR (short wave radiometer) | Volts |
| 13 | PAR (photo-synthetically available radiation, 400 - 700 nm) | Volts |

## ltsg

02+310:23:57:30.200        8542    -1.2580    34.1740    -1.2030      3.435      0.349  27.361205

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | Scan number |  |
| 3 | Internal water temperature | C |
| 4 | Salinity | PSU |
| 5 | External water temperature | C |
| 6 | Transmissometer signal | Volts |
| 7 | Fluorometer signal (analog) | Volts |
| 8 | Conductivity | S/cm |

## lpco

02+319:23:59:13.748 2002319.99851    7154.27     26.49    1033.6     325.79      6.74     329.3   53.76    0   Equil

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | Julian date file string | Julian |
| 3 | IR voltage reading | mV |
| 4 | Cell temperature | C |
| 5 | Barometer | millibars |
| 6 | VCO2 | mL |
| 7 | Equilibrator temperature | C |
| 8 | PCO2 | millibars |
| 9 | Gas flow | mL/min |
| 10 | Solenoid position ID | number |
| 11 | Measured gas | name |

## svp1

00+348:01:59:52.128  1539.40

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | Sound velocity | m/s |

## ladc

00+019:23:59:59.099 $PUHAW,UVH,-1.48,-0.51,250.6

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | $PUHAW |  |
| 3 | UVH (E-W, N-S, Heading) |  |
| 4 | Ship Speed relative to reference layer1 velocity2, East vector | knots |
| 5 | Ship Speed relative to reference layer1 velocity2, North vector | knots |
| 6 | Ship heading | degrees |

1The reference layer is an average velocity measured in a number of depth “bins”. On the LMG, the bins are eight meters deep and bins 3-10 define the reference layer. Hence, the reference layer is the water column from 16-80 meters beneath the ship.

2The speed ouput is water velocity relative to the ship’s hull and is therefore opposite of the actual

movement of the ship. For example, if the ship’s heading is due north, the North/South reference layer

velocity is likely to be negative (southerly).

## lash

##### ATTD: Attitude Data

01+081:00:00:00.806 $PASHR,ATT,345605.0,165.03,+001.86,-01.96,0.0018,0.0173,0*22

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $PASHR |  |
| 2 | ATT |  |
| 3 | GPS Time sec. of the week | seconds |
| 4 | heading (rel. to true North) | degrees |
| 5 | pitch | degrees |
| 6 | roll | degrees |
| 7 | Measurement RMS error | meters |
| 8 | Baseline RMS error | meters |
| 9 | attitude reset flag |  |

01+081:00:00:00.966 $GPGGA,235952.00,6051.7937,S,06030.2175,W,1,08,01.0,+00068,M,,M,,*79

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPGGA |  |
| 2 | UTC time at position | hhmmss.ss |
| 3 | Latitude | ddmm.mmm |
| 4 | North (N) or South (S) |  |
| 5 | Longitude | ddmm.mmm |
| 6 | East (E) or West (W) |  |
| 7 | GPS quality (1=GPS 2=DGPS) |  |
| 8 | Number of GPS satellites used |  |
| 9 | HDOP |  |
| 10 | Antenna Height | meters |
| 11 | M for Meters |  |
| 12 | Geoidal height | meters |
| 13 | M for meters |  |
| 14 | age of diff. GPS data | sss |
| 15 | differential reference station ID | aaaa |

## lgyr

02+315:23:59:58.194 $PASVW,00.1,A*1D

02+315:23:59:58.414 $IIVHW,287.7,T,,M,,N,,K*71

02+315:23:59:58.616 $HEHDT,287.7,T*25

02+315:23:59:58.821 $HEROT,001.6,A*2C

02+315:23:59:58.984 $HCHDT,,T*07

##### HDT: True Heading

01+083:00:00:02.893 $HEHDT,246.3,T*2C

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $HEHDT |  |
| 2 | Heading XXXXX = ddd.d | degrees |
| 3 | T flag for true heading, checksum |  |

##### ROT: Rate of Turn

01+083:00:00:03.093 $HEROT,-006.3,A*03

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $HEROT |  |
| 2 | Rate of turn | degrees/min |
| 3 | Status: A = data valid, checksum |  |

## tgps

##### GGA: Global Positioning Fix Data

00+040:00:00:00.985 $GPGGA,000003,6139.961,S,05949.422,W,1,6,001.64,-00036,M,00000,M,,

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPGGA |  |
| 2 | Latitude in degrees with decimal minutes | ddmm.mmm |
| 3 | North (N) or South (S) |  |
| 4 | Longitude in degrees with decimal minutes | ddmm.mmm |
| 5 | East (E) or West (W) |  |
| 6 | GPS quality (1=GPS 2=DGPS) |  |
| 7 | Number of GPS satellites used |  |
| 8 | Horizontal dilution of precision (HDOP) |  |
| 9 | Antenna height above/below mean-sea-level (geoid) | meters |
| 10 | Units for antenna height (M = Meters) |  |
| 11 | Geoidal Separation1 |  |
| 12 | Units for Geoidal Separation (M = Meters) | meters |
| 13 | Age of differential GPS data, number of seconds since last SC104 Type 1 or 9 |  |
| 14 | Differental reference station ID |  |

1Geoidal Separation: the difference between the WGS-84 earth ellipsoid and mean-sea-level (geoid). A negative value represents mean-sea-level below ellipsoid.

##### GLL: Geographic Position – Latitiude/Logitude

00+040:00:00:00.065 $GPGLL,6139.96,S,05949.42,W,000002,A

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPGLL |  |
| 2 | Latitude | ddmm.mmm |
| 3 | North (N) or South (S) |  |
| 4 | Logitude | ddmm.mmm |
| 5 | East (E) or West (W) |  |
| 6 | UTC of position | hhmmss.ss |
| 7 | Status: A = Data Valid |  |

##### VTG: Track Made Good and Speed over Ground

00+040:00:00:00.213 $GPVTG,161,T,149,M,009.6,N,017.8,K

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPVTG |  |
| 2 | Track, degrees true | degrees |
| 3 | T flag for True |  |
| 4 | Track, degrees magnetic | degrees |
| 5 | M flag for Magnetic |  |
| 6 | Speed over Ground | knots |
| 7 | N flag for Knots |  |
| 8 | Speed over Ground | kmhr |
| 9 | K flag for km/hr |  |

##### VHW: Speed Through Water and Heading

00+040:00:00:00.212 $GPVHW,246,T,234,M,012.3,N,022.8,K

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPVHW |  |
| 2 | Heading, degrees True | degrees |
| 3 | T flag for True |  |
| 4 | Heading, degrees Magnetic | degrees |
| 5 | M flag for Magnetic |  |
| 6 | Speed through water | knots |
| 7 | N flag for Knots |  |
| 8 | Speed through water | km/hr |
| 9 | K flag for km/hr |  |

##### ZDA: Time and Date

00+040:00:00:00.285 $GPZDA,000002,09,02,2000,00,00

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPZDA |  |
| 2 | UTC time | hhmmss.ss |
| 3 | Day: 01 – 31 | dd |
| 4 | Month: 01 – 12 | mm |
| 5 | Year | yyy |
| 6 | Local time zone description1, 00 +/-13 hrs |  |
| 7 | Local time zone minutes description, same sign as local hours |  |

1Zone description is the number of whole hours added to local time to obtain GMT, values are negative for East longitudes.

##### BWC: Bearing and Distance to Waypoint

00+040:00:00:00.865 $GPBWC,000003,6209.70,S,05824.00,W,127.2,T,115.3,M,050.1,N,014

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPBWC |  |
| 2 | UTC of bearing | hhmmss.ss |
| 3 | Destination waypoint latitude in degrees, decimal minutes | ddmm.mmm |
| 4 | Hemisphere Flag: N or S |  |
| 5 | Destination waypoint longitude in degrees, decimal minutes | ddmm.mmm |
| 6 | Hemisphere Flag: E or W |  |
| 7 | Bearing, degrees true | degrees |
| 8 | T flag for True |  |
| 9 | Bearing, degrees magnetic | degrees |
| 10 | M flag for Magnetic |  |
| 11 | Distance to waypoint in nautical miles | nm |
| 12 | N flag for Nautical Miles |  |
| 13 | Waypoint ID |  |

## lpcd

##### GGA: GPS Position Fix – Geoid/Ellipsoid

00+019:23:59:59.301 $GPGGA,235958.409,6849.6944,S,13712.8472,W,1,06,1.2,092.4,M,047.3,M,,*67

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPGGA |  |
| 2 | UTC time at position | hhmmss.sss |
| 3 | Latitude | ddmm.mmm |
| 4 | North (N) or South (S) |  |
| 5 | Longitude | ddmm.mmm |
| 6 | East (E) or West (W) |  |
| 7 | GPS quality (1=GPS 2=DGPS 3=P-CODE) |  |
| 8 | Number of GPS satellites used |  |
| 9 | HDOP |  |
| 10 | Antenna Height | meters |
| 11 | M for Meters |  |
| 12 | Geoidal height | meters |
| 13 | M for meters |  |
| 14 | Null field |  |
| 15 | Checksum |  |

##### GLL: GPS Latitude/Longitude

00+019:23:59:59.381 $GPGLL,6849.6944,S,13712.8472,W,235958.409,A*35

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPGLL |  |
| 2 | Latitude | degrees |
| 3 | North or South |  |
| 4 | Longitude | degrees |
| 5 | East or West |  |
| 6 | UTC of position | hhmmss.sss |
| 7 | staus of data (A = valid) |  |
| 8 | Checksum |  |

##### VTG: GPS Track and Ground Speed

00+019:23:59:59.382 $GPVTG,238.7,T,182.3,M,001.8,N,003.3,K*41

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPVTG |  |
| 2 | Heading | degrees |
| 3 | degrees True (T) |  |
| 4 | Heading | degrees |
| 5 | degrees magnetic (M) |  |
| 6 | Ship speed | knots |
| 7 | N = knots |  |
| 8 | Speed | km/hr |
| 9 | Checksum |  |

# LMG Sensors

## Shipboard Sensors

| Sensor | Description | Serial # | Cal. Date | Status |
| --- | --- | --- | --- | --- |
| Port Anemometer | R.M. Young 105106 | WM35061 | 07/28/02 | collected |
| Stbd Anemometer | R.M. Young 105106 | WM28393 | 07/28/02 | collected |
| Barometer | R.M. Young 61201 | BP00873 | 08/15/01 | collected |
| Humidity/Wet Temp | R.M. Young 41372VC | 6133 | 09/13/02 | collected |
| Mast PAR | BSI QSR-240 | 6394 | 06/05/01 | collected |
| Pyranometer | Eppley PSP | 31701F3 | 09/18/02 | collected |
| Pyrgeometer | Eppley PIR | 32031F3 | 09/18/02 | collected |
| GUV | Biospherical GUV-511 | 9228 | 11/03/01 | Not Collected |
| TSG | SeaBird SBE21 | 1789 | 08/02/02 | collected |
| TSG Remote Temp | SeaBird 3-01/S | 1619 | 09/13/02 | collected |
| Fluorometer | Turner 10-AU-005 Lamp: daylight 10-045, reference filter: 10-052, emission filter: 10-051, excitation filter: 10-050. | 6046RTD | n/a | collected |
| Transmissometer | WET Labs 9707017 | CST-424R | 07/31/02 | collected |
| P-Code GPS | Trimble 20636-00 (SM) | 220035265 | n/a | collected |
| Bathymetry | Knudsen 320B/R |  | n/a | collected |

## CTD Sensors

| Sensor | Description | Serial # | Cal. Date |
| --- | --- | --- | --- |
| CTD Fish | Sea-Bird 9Plus-3400m | 091480 | 29-Jun-01 |
| CTD Deck Unit | Sea-Bird 11Plus | 288 | n/a |
| Prim. Temp. Sensor | Sea-Bird 3-02/F | 1542 | 23-Jul-02 |
| Sec. Temp. Sensor | Sea-Bird 3-02/F | 2205 | 25-Jul-02 |
| Prim. Cond. Sensor | Sea-Bird 4-02/0 | 1223 | 25-Jun-02 |
| Sec. Cond. Sensor | Sea-Bird 4C | 1200 | 06-Aug-02 |
| Diss. Oxygen Sensor | Sea-Bird SBE43 | 0200 | 02-Dec-02 |
| Fluorometer | Chelsea model Mk III Aquatracka | 088015 | 8-Aug-02 |
| Transmissometer | C-Star Transmissometer | CST-248DR | 31-Jul-02 |

# Acquisition and Processing Information

## Processing Specifics

Refer to the instrmnt.cof file along with the specific instrument calibration sheets, both located in the Cal/ directory of the data distribution, for information on how the RVDAS data was collected and processed.

## Errors and Events

This section lists all significant events and known problems with acquisition during this cruise including instrument failures, data acquisition system failures, and other factors affecting this data set.

| Date (Julian) | Time (GMT) | Event | Location |
| --- | --- | --- | --- |
| 044 | 03:24 | START DAS | PA DOCK |
| 044 | 03:25 | START SONAR | PA DOCK |
| 044 | 13:45 | START ADCP | 68 West |
| 044 | 15:05 | Start TSG & PCO2 | 68 West |
| 045 | 18:32 | OCEANUS REBOOTED |  |
| 047 | 00:58 | REBOOTED PONTUS, OCEANUS and CYCLONE |  |
| 048 | 03:29 | SHUTDOWN PONTUS, OCEANUS and CYCLONE |  |
| 048 | 04:33 | RESTART PONTUS, OCEANUS and CYCLONE |  |
| 048 | 11:57 | Turned off Seawater flow, PCO2, Sonar and TSG | Arrived Palmer Station |
| 048 | 16:42 | Change ADCP Pulse width from 16 to 8 | At Palmer Station |
| 048 | 18:00 | Turned on Seawater flow, PCO2, Sonar and TSG | Depart Palmer Station |
| 048 | 21:34 | Change Sound speed on Sonar to 1456 from 1500 |  |
| 055 | 19:33 | Cycled ADCP and GENTOO to fix ASHTECH failure |  |
| 061 | 11:01 | Turned off Seawater flow, PCO2, Sonar and TSG | Arrived Palmer Station |
| 061 | 11:02 | Change ADCP Pulse width from 8 to 16 | At Palmer Station |
| 062 | 13:15 | Turned on Seawater flow, PCO2, Sonar and TSG | Depart Palmer Station |
| 065 | 21:40 | Stop TSG, ADCP, PCO2 | 68 West |
| 065 | 21:50 | Stop DAS | 68 West |