Data Report

NBP0104

GLOBEC II

July 22 – August 31, 2001

Prepared by: E. Joynt, T. Bolmer, and K. Gavahan

Introduction	1

Archive Data Extraction	2

Distribution Contents	3

ADCP	3

CTD	3

Cruise Tracks	4

GUV	4

Satellite Images	5

Science Report	5

NBP Data Products: MGD77 & JGOFS	5

JGOFS	5

MGD77	6

RVDAS	7

Data File Names and Structures	8

Ocean Data Files	14

Calculations	15

TSG	15

PAR	15

PIR	15

PSP	15

NBP0104 Sensors	16

Shipboard Sensors	16

NBP0104 CTD Sensors:	16

NBP0104 MOCNESS Sensors:	16

Acquisition Problems and Events	17

Calibrations	18

TSG CALIBRATION FILES	18

Conductivity Sensor	18

Temperature	19

Remote Temperature	20

Transmissometer	21

Meteorology System	22

Windbirds	22

PSP	25

PIR	26

GUV	27

CTD	29

Pressure Sensor	29

Temperature Sensors	30

Conductivity Sensors	32

Dissolved Oxygen Sensor	34

Fluorometer	35

CTD PAR	37

CTD Transmissometer	38

10 m2 MOCNESS	39

Pressure Sensor	39

Temperature Sensor	40

Conductivity Sensor	41

1 m2 MOCNESS	42

Pressure Sensor	42

Temperature Sensor	43

Conductivity	44

Transmissometer	45

Fluorometer	46

Optical Plankton Counter	47

# Introduction

The NBP data acquisition systems continuously log data from several instruments throughout the cruise.  This document describes the format of that data and its location on the distribution media.  It also contains important information which may affect how this data is processed such as instrument failures or other known problems with acquisition.

The data collected during this cruise is distributed on a CD-ROM written in ISO9660 level-1 format.  This data format has very strict requirements on filenames and organization.  However, it is readable by virtually every computing platform.

The data is contained in a Unix tar archives (.tar extension).  All of the data has been compressed using Unix “gzip” compression.  Gzipped files have a “.gz” extension.  Tools are available on all platforms for uncompressing and de-archiving these formats.  On Macintosh, Stuffit Expander with DropStuff will open a tar archive and uncompress gzipped and Unix compressed files.  For Windows, WinZip, a shareware utility included on this CD (remember, it is shareware) will open these files.

IMPORTANT:  Read the last section in this document, Acquisition Problems and Events, for important information that may affect the processing of this data.

# Archive Data Extraction

It is often useful to know exactly how an archive was produced when expanding its contents.  The tar files on the CD  were created on an SGI using the following commands:

tar cvLf archive-file files-to-be-archived

To create a list of the files in the archive:

tar tvf  archive-file  > contents.list

To extract the files from the archive:

tar xvf  archive-file file(s)-to-extract

G-zipped files will have a “.gz” extension on the filename. These files can be decompressed after de-archiving, using:

gunzip filename.gz

The directories in the archive are structured in the following manner:

| Volume 1 | Volume 2 |
| --- | --- |
| geopdata/ 0104jgof.tar 0104mgd.tar interim process files NBP0104.trk NBP0104.mgd NBP0104.gmt other/ 0104ess.tar 0104guv.tar 0104ice.tar 0104moc.tar 0104nutr.tar 0104opc.tar 0104snby.tar plots/ Cruise Plots report/ Data Report rvdas/uw/ 0104bat.tar 0104flr.tar 0104grv.tar 0104met.tar 0104pco2.tar 0104sim.tar 0104svp.tar 0104tsg.tar SciRep/Daily Cruise Reports Utility/ WInZip | imagery/ TeraScan satellite images ocean/ 0104ctd.tar 0104pco2.tar 0104tsgf.tar 0104xbt.tar rvdas/nav/01043df.tar 0104adcp.tar 0104gyr.tar 0104ngl1.tar 0104PCOD.tar seabeam/ daily plots survey plots |

# Distribution Contents

## ADCP

ADCP data during this cruise included additional data files that are not normally collected by this system.  The additional size of these files necessitated distribution of the ADCP data on a separate CD set.  Please refer to that CD data set.

## CTD

The ctd data and report have been placed in the tar file 0104ctd.tar, which contains the flowing structure:

| ctd.list (list of all ctd stations) ctdsetup/ (batch files, cfg & con files) data/ (raw datafiles) plots/ (up & down trace postscript plots) seasoft4.234/ (application for processing ctd data) | report/section/ (section plots) casts/ (plots of individual casts) text/ (ctd data report) seacat/ (data structure for SeaCat ctd) |
| --- | --- |

Individual CTD casts are represented by a set of four files containing a bottle-firing file (.bl), a configuration file (.con), a data file (.dat) and a header file (.hdr).   Casts are numbered according to the cruise id number (0104) followed by the number of the cast.  For example; the raw files associated with the third cast on this cruise are: 0104003.bl, 0104003.con, 0104003.dat, 0104003.hdr.  The raw and processed data files are in binary format.  The 1 db bin averaged up and down traces have been converted to ascii (.asc files).

SeaBird’s SeaSoft software used to acquire the data is included in the CTD data distribution in the “Seasoft” directory.  SeaSoft is a DOS-based software package, but can be run in a DOS window under the Windows9X operating systems for cast playback and data analysis. The software package used to process this data (version 4.234) is included on this CD in the directory Seasoft.  The configuration files and processing scripts (written by Suzanne O’Hara for the standard processing of the SBE 9/11plus) are also included in the Seasoft directory under in the ctdsetup directory.  The directory report contains the CTD data report with folder for all plots produced during the cruise. The directory seacat gas a structure similar to the ctd directory and contains the data from the SeaCat CTD unit.

File extension definitions:

| EXT | Description |
| --- | --- |
| ASC | The data portion of a .CNV converted data file written in ASCII by ASCIIOUT, or files written by TERM37. |
| BL | Created by SEASAVE when a bottle fire confirmation is received. Contains bottle sequence number, position, date, time, beginning and ending scan numbers. |
| BTL | Created by ROSSUM. This is a summary of the data in a .ROS file. |
| BSR | Bottle scan range file, used by DATCNV to create a .ROS file. |
| CFG | Used by SEASOFT modules to store the input filename, input data path, output data path, and other miscellaneous module specific parameters. |
| CTR | Density contour file generated by CONTOUR. |
| CNV | ‘Converted’ engineering unit data file. An ASCII header precedes the data. |
| CON | Contains instrument configuration and calibration coefficients, used by SEACON, SEASAVE, and DATCNV |
| DAT | Raw binary data, optionally with header information (SBE 9/11, 11X, 9/11plus, and data files created with previous versions of SEASOFT). |
| DSP | Used by SEASAVE to store data acquisition and display parameters. |
| HDR | - Header portion of a .CNV converted data file written by ASCIIOUT. 2) Header recorded when acquiring real time data or uploading archived data. |
| HEX | Raw HEX data with header information (SBE 16, 17, 19, 21, and 25) |
| MRK | Marker file created by SEASAVE during real time data acquisition. |
| PLT | Used by SEAPLOT to store display parameters |
| ROS | Scans marked with the bottle fire confirmation bit, or defined by a .BSR file, written by DATCNV. |

*Note:  This is a complete list of all file extensions.  This data set may not contain ALL of the above extension files.

SEASOFT modules search the current directory for DSP, PLT, and CFG files.  SEASOFT modules search the ‘input data path’ for CON, HEX, DAT, and CNV files.  One exception is SEACON which searches the current directory for CON files.

For more information and updated software visit the web site at www.seabird.com; or contact (206) 643-9866, seabird@seabird.com, Sea-Bird Electronics 1808 – 136th Place NE Bellevue, WA 98005

## Cruise Tracks

/plots

PostScript cruise track and station plots have been produced for this cruise:

- Cast locations:  0104casts.ps

- Tow Locations: 0104tows.ps

- Cruise Track:  0104trk.ps

- Letter size, all stations: 0104all.ps

- Letter Size, bathymetry: bathsmall.ps

- Poster size, all stations: posterall.ps

- Poster size, bathymetry: posterbat.ps

## GUV

All files are placed in two zip-archives named puv.zip and guv.zip. The contents of the archives are in the files puvlst.txt and guvlst.txt respectively. GUV data was acquired continuously from the instrument mounted on the top of the science mast until the end of data collection fo rthe cruise. PUV data consists of a single cast on most days.  A copy of the PUV/GUV calibration file used for Biospherical Instruments’ PUV500 acquisition software is included in the puvguv directory on CD1 as calibr8.puv.

The PUV files were converted to CSV (Comma Separated Value) format using the Biospherical Instruments’ PUVLOG program. Similarly, the GUV files were converted to CSV format using the Biospherical Instruments’ PUVPROF program.  For additional information on the PUV/GUV data collected during NBP9901, contact:

Dr. Chris Fritzen, at  cfritsen@dri.edu cfritsen@dri.edu or at: University and Community College System of Nevada, Desert Research Institute, Biological Sciences Center, 7010 Dandini Blvd, Reno, NV 89512, USA (715) 673-7487.

The columnar format of the CSV file is listed below:

| Field | PUV Data |  | Field | GUV Data |
| --- | --- | --- | --- | --- |
| 1 | Depth |  | 1 | Depth |
| 2 | 308 U |  | 2 | 306 U |
| 3 | 320 U |  | 3 | 320 U |
| 4 | Temp U |  | 4 | Temp U |
| 5 | MATFL |  | 5 | MATFL |
| 6 | 340 U |  | 6 | 340 U |
| 7 | 380 U |  | 7 | 380 U |
| 8 | PAR U |  | 8 | PAR U |
| 9 | Temp U |  | 9 | Temp S |
| 10 | 308 S |  | 10 | 300 S |
| 11 | Batt S |  | 11 | Batt S |
| 12 | 320 |  | 12 | 320 S |
| 13 | 340 |  | 13 | 340 S |
| 14 | 380 |  | 14 | 380 S |
| 15 | Gnd |  | 15 | Gnd S |
| 16 | PAR |  | 16 | PAR S |
| 17 | K308 |  |  |  |
| 18 | K320 |  |  |  |
| 19 | K340 |  |  |  |
| 20 | K380 |  |  |  |
| 21 | Kpar |  |  |  |
| 22 | Chl |  |  |  |
| 23 | Prod |  |  |  |

## Satellite Images

/imagery

Satellite Images processed for this cruise were organized into three folders cwifs, ice and wx for the different types of TeraScan products.  Files were named in the following convention:

IDJJJYYA.jpg where:

ID = imge type (is = ice ssmi, iv = ice visible, cw = seawifs, wx = weather)

JJJ = Julian day

YY = year

A = used to allow for multiple images of type for that day

## Science Report

/reports

The daily science reports in html format.

## NBP Data Products: MGD77 & JGOFS

NBP0104.mgd

NBP0104.gmt

/geopdata/0104jgof.tar

/geopdata/0104mgd.tar

Two data products are created on each cruise of the NBP: JGOFS and MGD77.   Also, included in the PROC directory are interim process files of some of the data.  These are useful if it is necessary to reporcess the data through the RVDAS daily processing routines.

### JGOFS

The JGOFS data set consists of a single file produced each day named jgDDD.dat.gz where DDD is the Julian day the data was acquired. The “.gz” extension indicates that the individual files are compressed before archiving. The daily file consists of 20 separate columnar fields in text format, which are described below. The JGOFS data set is obtained primarily by applying calibrations to raw data and decimating to whole minute intervals.  However, several fields are derived measurements from more than a single raw input.  For example, Course Made Good (CMG) and Speed Over Ground (SOG) are calculated from gyro and GPS inputs by the NGL software package.  Similarly, the wind direction field is the vector sum of the separate X and Y inputs received from the wind instrument. The JGOFS data set was used to produce the daily data plots during the cruise.  Note: Null, unused, or unknown fields are filled with 9’s in the JGOFS data.  TSG data is processed by RVDAS.

| Field | Data | Units |
| --- | --- | --- |
| 01 | GMT date | dd/mm/yy |
| 02 | GMT time | hh:mm:ss |
| 03 | NGL latitude (negative is South) | dd.dddd |
| 04 | NGL longitude (negative is West) | ddd.dddd |
| 05 | Speed over ground | Knots |
| 06 | GPS HDOP | - |
| 07 | Gyro Heading | Degrees (azimuth) |
| 08 | Course made good | Degrees (azimuth) |
| 09 | mast PAR | Einsteins/meters2 sec |
| 10 | sea surface temperature | C |
| 11 | sea surface conductivity | siemens/meter |
| 12 | sea surface salinity | PSU |
| 13 | sea depth (uncorrected, calc. sw sound vel. 1500 m/s) | meters |
| 14 | true wind speed (port windbird) | meters/sec |
| 15 | true wind direction (port windbird) | degrees (azimuth) |
| 16 | Ambient air temperature | C |
| 17 | Relative humidity | % |
| 18 | Barometric pressure | mBars |
| 19 | sea surface fluorometry | volts (0-5 FSO) |
| 20 | not used | - |
| 21 | PSP | W/m2 |
| 22 | PIR | W/m2 |

### MGD77

The MGD77 data set is contained in a single file for the entire cruise named NBP0104.mgd, There is also a file named NBP0104.gmt.  This file is the output of the mgd77togmt utility using NBP0104.mgd as input.  The “gmt” file can be useful for plotting data using the GMT plotting package.  The directory /geopdata/PROC contains a file from each day of data acquisition named: Dddd.fnl.gz, where ddd is the Julian day.  These files contain all the data used to produce the “mgd” file, but in a space-delimited columnar format that may be more accessible for some purposes. In addition, these files contain data at one-second intervals rather than one minute and are individually “gzipped” to save space. Below is a detailed description of the MGD77 data set format.

All decimal points are implied. Leading zeros and blanks are equivalent.  Unknown or unused fields are to be filled with 9’s.  All “corrections”, such as time zone, diurnal magnetics, and Eotvos, are understood to be added.

| Col | Len | Type | Description |
| --- | --- | --- | --- |
| 1 | 1 | int | DATA RECORD TYPE Set to “3” for data record. |
| 2-9 | 8 | char | SURVEY IDENTIFIER |
| 10-14 | 5 | int | TIME-ZONE CORRECTION: In hundredths of hours. Corrects time (in characters 13-27) to GMT when added: equals zero when time is GMT. |
| 15-16 | 2 | int | YEAR 2 digit year |
| 17-18 | 2 | int | MONTH (e.g. May is represented as 05) |
| 19-20 | 2 | int | DAY Day of month |
| 21-22 | 2 | int | HOUR |
| 23-27 | 5 | real | MINUTES X 1000 |
| 28-35 | 8 | real | LATITUDE X 100000 + = North; - = South. (–9000000 to 9000000) |
| 36-44 | 9 | real | LONGITUDE X 100000 + = East; - = West. (–18000000 to 18000000) |
| 45 | 1 | int | POSITION TYPE CODE: 1=Observed fix; 3=Interpolated; 9=Unspecified |
| 46-51 | 6 | real | BATHYMETRY, 2- WAY TRAVELTIME: In 10,000th of seconds. Corrected for transducer depth and other such corrections |
| 52-57 | 6 | real | BATHYMETRY, CORRECTED DEPTH: In tenths of meters. |
| 58-59 | 2 | int | BATHYMETRIC CORRECTION CODE: This code details the procedure used for determining the sound velocity correction to depth |
| 60 | 1 | int | BATHYMETRIC TYPE CODE: 1 = Observed; 3 = Interpolated (Header Seq. 12); 9 = Unspecified |
| 61-66 | 6 | real | MAGNETICS TOTAL FIELD, 1ST SENSOR: In tenths of nanoteslas (gammas). |
| 67-72 | 6 | real | MAGNETICS TOTAL FIELD, 2ND SENSOR: In tenths of nanoteslas (gammas). For trailing sensor. |
| 73-78 | 6 | real | MAGNETICS RESIDUAL FIELD: In tenths of nanoteslas (gammas). The reference field used is in Header Seq. 13. |
| 79 | 1 | int | SENSOR FOR RESIDUAL FIELD 1 = 1st or leading sensor; 2 = 2nd or trailing sensor; 9 = Unspecified |
| 80-84 | 5 | real | MAGNETICS DIURNAL CORRECTION: In tenths of nanoteslas (gammas). (In nanoteslas) if 9-filled (i.e., set to “+9999”), total and residual fields are assumed to be uncorrected; if used, total and residuals are assumed to have been already corrected. |
| 85-90 | 6 | F6.0 | DEPTH OR ALTITUDE OF MAGNETICS SENSOR: In meters. + = Below sea level 3 = Above sea level |
| 91-9 | 7 | real | OBSERVED GRAVITY: In 10th of mgals. Corrected for Eotvos, drift, tares. |
| 98-10 | 6 | real | EOTVOS CORRECTION: In tenths of mgals. E = 7.5 V cos phi sin alpha + 0.0042 V*V |
| 104-108 | 5 | real | FREE-AIR ANOMALY In tenths of milligals Free-air Anomaly = G(observed) – G(theoretical) |
| 109-113 | 5 | char | SEISMIC LINE NUMBER: Used for cross-referencing with seismic data. |
| 114-119 | 6 | char | SEISMIC SHOT-POINT NUMBER |
| 120 | 1 | int | QUALITY CODE FOR NAVIGATION: 5=Suspected, by the originating institution;6=Suspected, by the data center, 9=No identifiable problem found |

## RVDAS

/rvdas/uw

/rvdas/nav

RVDAS (Research Vessel Data Acquisition System) was developed at Lamont-Doherty Earth Observatory of Columbia University and has been used on the R/V Maurice Ewing for several years.  It was adapted for use on the Nathaniel B. Palmer and her sister ship, the R/V Laurence M. Gould.

Daily data processing of the RVDAS data is performed to convert values into uasable units and as a check of the proper operation of the DAS. Both the raw and processed data sets from RVDAS are included in the data distribution. Below you will find detailed information on the data included. Be sure to read the “Significant Acquisition Events” section below for important information about data acquisition during this cruise.

#### Meteorological and Light Data

| Measurement | File ID | Collect. Status | Rate | Instrument |
| --- | --- | --- | --- | --- |
| Air Temperature | met1 | continuous | 1 sec | R. M. young 41372LC/LF |
| Relative Humidity | met1 | continuous | 1 sec |  |
| Wind Speed/Direction | met1 | continuous | 1 sec | R.M. Young 05106 |
| Barometer | bar1 | continuous | 1 sec | R.M. Young 61201 |
| PIR (LW radiation) | met1 | continuous | 1 sec | Eppley PIR |
| PSP (SW radiation) | met1 | continuous | 1 sec | Eppley PSP |
| PAR | met1 | continuous | 1 sec | BSI QSR-240 |
| GUV |  | continuous |  | BSI PUV-511A |

#### Navigational Data

| Measurement | File ID | Collect. Status | Rate | Instrument |
| --- | --- | --- | --- | --- |
| Attitude GPS | 3df1 | continuous | 1 sec | Ashtec 12 |
| P-Code GPS | PCOD | continuous | 1 sec | Trimble 20636-00SM |
| Gyro | gyr1 | continuous | 0.2 sec | Yokogawa Gyro |
| NGL | ngl1 | continuous | 1 sec | NGL Processed Nav Data |

#### Geophysical Data

| Measurement | File ID | Collect. Status | Rate | Instrument |
| --- | --- | --- | --- | --- |
| Gravimeter | grv1 | continuous | 10 sec | Lacoste & Romberg Gravity |
| Magnetometer | mag1 | not collected | 15 sec | EG&G G-866 |
| Bathymetry | bat1 | continuous | Varies | ODEC Bathy 2000 |
| Bathymetry | sim1 | depth < 2500 m | Varies | Simrad EK200 Sonar |
| Bathymetry | knu1 | not collected | varies | Knudsen |

#### Oceanographic Data

| Measurement | File ID | Collect. Status | Rate | Instrument |
| --- | --- | --- | --- | --- |
| Conductivty | tsg1 | continuous | 6 sec | SeaBird 21 |
| Salinity | tsgfl | continuous | 6 sec | calculated PSS78 |
| Sea Surf. Temperature | tsg1 | continuous | 6 sec | SeaBird 3-01/S |
| Fluorometry | flr1 & tsg1 | continuous | 6 sec | Turner 10-AU-005 |
| Transmissometry | tsg1 | continuous | 6 sec | WET Lab C-Star |
| pCO2 | pco2 | continuous | 70 sec |  |
| ADCP | adcp | continuous | varies | RD Instruments |

### Data File Names and Structures

RVDAS data is divided into two broad categories, Underway and Navigation.  The groups are abbreviated “uw” and “nav”.  These two subdirectories exist under the top-level rvdas directory. The instruments are broken down as shown. Each data file is gzipped to save space on the distribution.  Not all data types are collected every day or on every cruise.

RVDAS data files are named following the convention: NBP[CruiseID][FileID].dDDD.

- The CruiseID is the numeric name of the cruise, for example: NBP0104.

- The FileID (aka Channel ID) is a 4-character code representing the system being logged, for example: met1 (for meteorology)

- DDD is the year day of the data collection

| Underway Data | Channel ID |  | Navigation Data | Channel ID |
| --- | --- | --- | --- | --- |
| Barometer | bar1 |  | Ashtech GPS | 3df1 |
| Bathy 2000 | bat1 |  | Trimble GPS (P-Code) | PCOD |
| Fluorometer | flr1 |  | Gyro Compass | gyr1 |
| Gravimeter | grv1 |  | Furuno GPS | gp02 |
| Magnetometer | mag1 |  | NGL | ngl1 |
| Meteorological | met1 |  | ADCP course | adcp |
| Simrad | sim1 |  |  |  |
| Thermosalinograph | tsg1 |  |  |  |
| PCO2 | pco2 |  |  |  |
| Knudsen | knud |  |  |  |

Data is received by the RVDAS system via RS-232 serial connections. The data files that comprise the rvdas data set are described below. A time tag is added to each line of data received and the data is written to disk.

YY+DDD:HH:MM:SS.SSS [data stream from instrument]

Where, YY: two-digit year, DDD: Julian Day, HH: 2 digit hours, MM: 2 digit minutes SS.SSS: seconds.  All times are UTC.

The delimters used to separate fields in the raw data files are usually spaces and commas, but other delimiters are used (::, =, @) and occasionally there is no delimiter.  Care should be taken when reprocessing the data that the field’s separations are clearly understood.  An example data

#### bat1

00+019:23:59:53.901 ;I04485.3ME-23.0,I00000.0,-99.9, 0000@01/11/00,23:59:52.08 PW2 PF1 SF1 PL3 MO4 SB3 PO0 TX1 TR: GM5 1500 06.7 -72.1

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | Flagged Low Freq. Chn. Depth w/ units ;FDDDDD.DUN F= V valid, I invalid | meters |
| 3 | Low Freq. Echo Strength EEE.EE | dB |
| 4 | Flagged High Freq. Chn. Depth – unused |  |
| 5 | High Freq. Echo Strength – unused |  |
| 6 | Signed Heave Data SHHHH | cm |
| 7 | Date | mm/dd/yy |
| 8 | Time | hh:mm:ss |
| 9 | transmit pulse window type: PW1 Rectangular, PW2 Hamming, PW3 Cosine, PW4 Blackman |  |
| 10 | Primary transmit frequency PF1 3.5 kHz, PF2 12.0 kHz |  |
| 11 | Parametric mode secondary freq. SF1 3.5 kHz, SF2 12.0 kHz |  |
| 12 | pulse length: PL1 200usec, PL2 500usec, PL3 1msec, PL4 2msec, PL5 5msec, PL6 10msec, PL7 25msec. If transmit mode is FM: PL1 25msec, PL2 50msec, PL3 100msec. |  |
| 13 | Operating Mode: MO1 CW parametric, MO2 CW, MO3 FM parametric, MO4 FM |  |
| 14 | Frequency sweep bandwidth: SB1 1 kHz, SB2 2 kHz, SB3 5 kHz |  |
| 15 | power level: PO1 0dB, PO2 -6dB, PO3 -12dB, PO4 -18dB, PO5 -24dB, PO6 -30dB, PO6 -30 dB, PO7 -36dB, PO8 -42dB |  |
| 16 | Transmit Mode: TX1 single ping active, TX2 pinger listen, TX3 multipinging TR, TX4 multipinging TR, TX5 multipinging TTRR, TX6 mulitpinging TTTTRRRR, TX7 mulitpinging TTTTTRRRRR |  |
| 17 | Transmit Rate: TR3 4Hz, TR4 2Hz, TR5 1Hz, TR6 .5Hz, TR7 .33Hz, TR8 .25Hz, TR9 .20Hz, TR: = .10Hz, TR; = .05Hz |  |
| 18 | System Gain Mode: GM0 hydrographic AGC, GM1 to GM9 hydrographic +3db to + 27db manual. GMA to GMD hydrographic + 30db through + 60db manual, GME to GMK sub-bottom 1 through sub-bottom 7 |  |
| 19 | speed of sound | m/sec |
| 20 | depth of sonar window below sea-level | meters |
| 21 | background noise level in fixed point reference | dB/V |

#### flr1

00+019:23:59:58.061 0  0818 ::  1/19/00 17:23:17 = 0.983  (RAW)   1.2 (C)

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | marker 0 to 8 |  |
| 3 | 4-digit index |  |
| 4 | date | mm/dd/yy |
| 5 | time | hh:mm:ss |
| 6 | signal |  |
| 7 | signal units of measurement |  |
| 8 | cell temperature |  |
| 9 | temperature units |  |

#### grv1

99+099:00:18:19.775 your_line#1999 99 01818 9735.4

| Field | Data | Units | Conversion |
| --- | --- | --- | --- |
| 1 | RVDAS Time Tag |  |  |
| 2 | text string |  |  |
| 3 | gravity device date | yyyyjjjhhmmss |  |
| 3 | gravity count | count | mgal = count x 1.0047 + offset |

#### knud

99+099:00:18:19.775 hf,305.2,lf,304.3

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | hf – high frequency flag (12 kHz) |  |
| 3 | high fequency depth | meters |
| 4 | lf – low frequency flag (3.5 kHz) |  |
| 5 | low frequency depth | meters |

#### mag1

99+099:00:00:23.203 %  0  98 235928    0?372453

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | % 0 denotes G-866 magnetometer |  |
| 3 | Julian Day |  |
| 4 | Time |  |
| 5 | 0? denotes high noise condition |  |
| 6 | Magnetic Data (last digit is 10th’s place) | nT |

#### met1

00+019:23:59:59.761 $MET:  0.84, 1.12, 0.76, 1.06, 4.98, 0.26, 1.49, 3.93, 8.94, 0.80, 0.01, 0.01, 0.01, 0.22, 0.02, 0.1,40.11,4.96

| Field | Data | Units | Conversion |
| --- | --- | --- | --- |
| 1 | RVDAS Time Tag |  |  |
| 2 | Port windbird Speed (relative) | m/s |  |
| 3 | Port windbird Direction (relative) | deg |  |
| 4 | Port windbird standard deviation | deg |  |
| 5 | Starboard windbird Speed (relative) | m/s |  |
| 6 | Starboard windbird Direction (relative) | deg |  |
| 7 | Starboard windbird standard deviation | deg |  |
| 8 | Barometer | mBar |  |
| 9 | Air Temperature | C |  |
| 10 | Relative Humidity | % |  |
| 11 | PSP (short wave radiation) | mV | see calc. section |
| 12 | PIR (long wave radiation) | mV | see calc. section |
| 13 | PAR (photosyntheticly available radiation) | mV | see calc section |

#### pCO2

00+021:23:59:43.190 2000021.9992 2382.4 984.2 30.73 50.8 345.9 334.1 -1.70 -68.046 -144.446 Equil

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | pCO2 Time Tag (decimal is time of day) | yyyjdd.fod |
| 3 | raw voltage | mV |
| 4 | barometer | mBar |
| 5 | cell temperature | C |
| 6 | flow rate | cm3/min |
| 7 | concentration | ppm |
| 8 | pCO2 pressure | microAtm |
| 9 | Equilibrated temperature | C |
| 10 | Latitude | degrees |
| 11 | Longitude | degrees |
| 12 | Flow Source (Equil = pCO2 measurement) |  |

#### sim1

00+005:00:00:52.388 D1,23583509,1479.6, 17,  1,  0

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | Header |  |
| 3 | Time Tag | hhmmss.sss |
| 4 | Depth | m |
| 5 | bottom surface backscattering strength | dBar |
| 6 | Transducer number ( 1 = 38 kHz ) |  |
| 7 |  |  |

#### stc1

00+019:23:59:46.976 ##ryyyydddhhmmssuuuuuuSSCC

00+347:00:00:05.013 ##BFFFF3470000049568311529

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | r is the input time code "B" for IRIG-B |  |
| 3 | yyyy is the year (FFFF if IRIG input) |  |
| 4 | ddd is the 3 digit day of the year |  |
| 5 | hh is 2 digit hour of day |  |
| 6 | mm is 2 digit minute of hour |  |
| 7 | ss is 2 digit second of minute |  |
| 8 | uuuuuu is 6 digits for microseconds digits |  |
| 9 | SS is 2 hex character DP-Extd_Sts dual port RAM value |  |
| 10 | CC is checksum |  |

#### svp1

00+348:01:59:52.128  1539.40

| Field |  | Data | Units |
| --- | --- | --- | --- |
| 1 | RVDAS Time Tag |  |  |
| 2 | Sound velocity |  | m/s |

#### syn1

00+120:10:46:30.665 *GCS900228     005   811.00001E00/04/29:10:45:5311060600000000403000-0.050.023    00000000-00101AP1N 000156498-0100002AP1N 00016350100100003AP1N 000162497-0200004AP1N 00015850000000005AP1N 00015550100100006AP1N 000157500000000

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | String tag |  |
| 3 | Number of bytes |  |
| 4 | Line number |  |
| 5 | Shotpoint number |  |

#### tsg1

00+019:23:59:46.976 15A16CFC163F8C2C100

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | Seabird Hex string (see notes on converting to real units) |  |

#### 3df1

##### PBEN: Measurement Data

00+019:23:59:57.054 $PASHR,PBN,345609.00,-1695527.0,-1569301.4,-5925126.0,-068:49.6968,-137:12.8448,00047.7,-000.69,000.67,-000.51,08,????,02,01,02,01*32

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $PASHR |  |
| 2 | PBN |  |
| 3 | GPS Time sec. of the week | seconds |
| 4 | Station Postion: ECEF X | meters |
| 5 | Station Postion: ECEF Y | meters |
| 6 | Station Postion: ECEF Z | meters |
| 7 | Latitude ( - = South ) | deg:min |
| 8 | Longitude ( - = West ) | deg:min |
| 9 | Altitude | meters |
| 10 | Velocity in ECEF X | m/sec |
| 11 | Velocity in ECEF Y | m/sec |
| 12 | Velofity in ECEF Z | m/sec |
| 13 | Number of satellites used |  |
| 14 | Site name |  |
| 15 | PDOP |  |
| 16 | HDOP |  |
| 17 | VDOP |  |
| 18 | TDOP |  |

##### ATTD: Attitude Data

00+019:23:59:57.854 $PASHR,ATT,345610.0,252.82,+000.52,+001.95,0.0011,0.0068,0

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $PASHR |  |
| 2 | ATT |  |
| 3 | GPS Time sec. of the week | seconds |
| 4 | Heading (rel. to true North) | degrees |
| 5 | Pitch | degrees |
| 6 | Roll | degrees |
| 7 | Measuremnet RMS error | meters |
| 8 | Baseline RMS error | meters |
| 9 | Attitude reset flag |  |

##### GGA: GPS Position Fix – Geoid/Ellipsoid

00+019:23:59:57.134 $GPGGA,235956.00,6849.6968,S,13712.8448,W,1,08,01.0,+00048,M,,M,,

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag $GPGGA |  |
| 2 | UTC time at position | hhmmss.ss |
| 3 | Latitude | ddmm.mmm |
| 4 | North (N) or South (S) |  |
| 5 | Longitude | ddmm.mmm |
| 6 | East (E) or West (W) |  |
| 7 | GPS quality (1=GPS 2=DGPS) |  |
| 8 | Number of GPS satellites used |  |
| 9 | HDOP |  |
| 10 | Antenna Height | meters |
| 11 | M for Meters |  |
| 12 | Geoidal height | meters |
| 13 | M for meters |  |
| 14 | Age of diff. GPS data |  |
| 15 | Differential reference station ID |  |

#### gyr1

00+019:23:59:59.952 $HEHRC25034,-020*73

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | $HEHRC |  |
| 3 | Heading XXXXX = ddd.dd | degrees |
| 4 | Rate of Change SYYY S = +/-, YYY = r.rr |  |
| 5 | Checksum |  |

#### ngl1

00+019:23:59:59.857 -68.82822,-137.21416,1.10,279.27,251.10,0.00,0.00,0,18.2587,1,1146973

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | Latitude (south is negative) | degrees |
| 3 | Longitude (west is negative) | degrees |
| 4 | Ship Speed | knots |
| 5 | Course made good | degrees |
| 6 | Gyro Heading | degrees |
| 7 | PDOP |  |
| 8 | HDOP |  |
| 9 | Quality |  |
| 10 | GPS up |  |
| 11 | Fix Number |  |

#### PCOD

##### GGA: GPS Position Fix – Geoid/Ellipsoid

00+019:23:59:59.301 $GPGGA,235958.409,6849.6944,S,13712.8472,W,1,06,1.2,092.4,M,047.3,M,,*67

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | $GPGGA |  |
| 3 | UTC time at position | hhmmss.sss |
| 4 | Latitude | ddmm.mmm |
| 5 | North (N) or South (S) |  |
| 6 | Longitude | ddmm.mmm |
| 7 | East (E) or West (W) |  |
| 8 | GPS quality (1=GPS 2=DGPS 3=P-CODE) |  |
| 9 | Number of GPS satellites used |  |
| 10 | HDOP |  |
| 11 | Antenna Height | meters |
| 12 | M for Meters |  |
| 13 | Geoidal height | meters |
| 14 | M for meters |  |
| 15 | Age of diff. GPS data |  |
| 16 | Differential reference station ID |  |
| 17 | Checksum |  |

##### GLL: GPS Latitude/Longitude

00+019:23:59:59.381 $GPGLL,6849.6944,S,13712.8472,W,235958.409,A*35

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | $GPGLL |  |
| 3 | Latitude | degrees |
| 4 | North or South |  |
| 5 | Longitude | degrees |
| 6 | East or West |  |
| 7 | UTC of position | hhmmss.sss |
| 8 | Staus of data (A = valid) |  |
| 9 | Checksum |  |

##### VTG: GPS Track and Ground Speed

00+019:23:59:59.382 $GPVTG,238.7,T,182.3,M,001.8,N,003.3,K*41

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | $GPVTG |  |
| 3 | Heading | degrees |
| 4 | Degrees True (T) |  |
| 5 | Heading | degrees |
| 6 | Degrees magnetic (M) |  |
| 7 | Ship speed | knots |
| 8 | N = knots |  |
| 9 | Speed | km/hr |
| 10 | K = km per hour |  |
| 11 | Checksum |  |

#### adcp

00+019:23:59:59.099 $PUHAW,UVH,-1.48,-0.51,250.6

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | $PUHAW |  |
| 3 | UVH (E-W, N-S, Heading) |  |
| 4 | Ship Speed relative to reference layer, East vector | kn |
| 5 | Ship Speed relative to reference layer, North vector | kn |
| 6 | Ship heading | degrres |

## Ocean Data Files

ocean/

#### pCO2-merged

00+346:23:58:20.672 2000346.9991 2398.4 1008.4 0.01 45.4 350.3 342.6 15.77 Equil -43.6826 173.1997 15.51 33.90 0.33 5.28 9.05 1007.57 40.0 14.87 182.44

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | PCO2 Time Tag (decimal is time of day) | yyyjdd.fod |
| 3 | raw voltage | mV |
| 4 | Barometer | mB |
| 5 | cell temperature | C |
| 6 | flow rate | cm3/min |
| 7 | Concentration | ppm |
| 8 | PCO2 pressure | microAtm |
| 9 | Equilibrated temperature | C |
| 10 | Flow Source (Equil = pCO2 measurement) |  |
| 11 | RVDAS latitude | degrees |
| 12 | RVDAS longitude | degrees |
| 13 | TSG external temperature | C |
| 14 | TSG salinity | PSU |
| 15 | TSG fluorometry | V |
| 16 | RVDAS true wind speed | m/s |
| 17 | RVDAS true wind direction | degrees |
| 18 | Barometric Pressure | Mbars |
| 19 | Uncontaminated seawater pump flow rate | l/min |
| 20 | Speed over ground | Knots |
| 21 | Course made good | Degrees |

#### tsgfl

00+075:00:00:04.467 -01.488 -01.720 02.6783 33.63748 1.002442 0.002442

| Field | Data | Units |
| --- | --- | --- |
| 1 | RVDAS Time Tag |  |
| 2 | Internal water temperature | C |
| 3 | Sea Surface Temperature | C |
| 4 | Conductivity | Siemens |
| 5 | Salinity | PSU |
| 6 | Fluorometry | V |
| 7 | Transmissometer | V |

#### xbt

0104xbt.tar

During the cruise Expendable Bathythermographs were used to obtain water column temperature profiles.  These were used to adjust the sound velocity profile for the SeaBeam system.  The data files from these launches are included.

# Calculations

## TSG

Raw TSG data is stored as a 20 byte (character) long hex string

| Bytes | Data |
| --- | --- |
| 1-4 | Sensor Temperature |
| 5-8 | Conductivity |
| 9-14 | Remote Temperature |
| 15-17 | Fluorometer voltage |
| 18-20 | Transmissometer voltage |

The coefficients for temperature and conductivity sensors can be found in the calibrations section..

Calculating Temperature – ITS-90

T = decimal equivalent of bytes 1-4

Temperature Frequency:  f = T/19 +2100

Temperature = 1/{g + h[ln(f0/f)] + i[ln2(f0/f)] + j[ln3(f0/f)]} - 273.15 (C)

Calculating Conductivity – ITS-90

C = decimal equivalent of bytes 5-8

Conductivity Frequency f = sqrt(C*2100+6250000)

Conductivity = (g + hf2 + if3 + jf4)/[10(1 + t + p)] (siemens/meter)

t = temperature (C); p = pressure (decibars);  = Ctcor;  = CPcor

Calculating Fluorometry Voltage

f = decimal equivalent of bytes 15-17

Fluorometry Voltage = f/819

Calculating Transmittance

Vdark = 0.058 V

Vref  = 4.765 V

t = decimal equivalent of bytes 18 – 20

Transmissometer Voltage (Vsignal) = t/819

% Transmittance = (Vsignal – Vdark) / (Vref - Vdark)

## PAR

raw data = mV

calibration scale = 6.08 V/(Einstiens/cm 2sec)

offset (Vdark) = 0.3 mV

(raw mV - Vdark)/scale x 104 cm2/m2 x 10-3 V/mV= Einstiens/m2sec

or

(data mV – 0.3 mV) x 1.65 (Einstiens/m2sec)/mV = Einstiens/m2sec

## PIR

raw data = mV

calibration scale = 4.13 x 10-6  V/(W/m2)

data mV / (scale x 103 mV/V ) = W/m2

or

data mV x 242.1(W/m2)/mV = W/m2

## PSP

raw data = mV

calibration scale =  8.28 x 10-6  V/(W/m2)

data mV / (scale x 103 mV/V) = W/m2

or

data mV x 120.7 (W/m2)/V = W/m2

# NBP0104 Sensors

## Shipboard Sensors

| Sensor | Description | Serial # | Cal. Date | Status |
| --- | --- | --- | --- | --- |
| Port Anemometer | RM Young 5106 | WM46262 | 4/11/01 | collect |
| Stbd Anemometer | RM Young 5106 | WM46263 | 4/11/01 | collect |
| Barometer | RM Young 61201 | 01705 |  | collect |
| Air Temp/Rel. Hum. | RM Young 41372LC | 06134 |  |  |
| Mast PRR | BSI PRR-610 |  |  | not collect |
| UW PRR | BSI PRR-600 |  |  | not collect |
| Mast PAR | BSI QSR-240 | 6356 | 2/15/01 | collect |
| Pyranometer | Eppley PSP | 33090F3 | 11/7/00 | collect |
| Pyrgeometer | Eppley PIR | 32845F3 | 2/22/01 | collect |
| P-Code GPS | Trimble 20636-00 (SM) |  |  | PCD/CIV |
| Attitude GPS | Ashtech 12 | 700273F2114 FW 7B13-D1-C21 |  | collect |
| TSG | SeaBird SBE21 | 214857-0857 | 2/7/01 | collect |
| TSG Remote Temp | SeaBird 3-01/S | 032593 | 1/3/01 | collect |
| Fluorometer | Turner 10-AU-005 Lamp: daylight 10-045; ref. filter: 10-052, em. filter: 10-051, ex. filter: 10-050 | 5651 FRTD |  | collect |
| Transmissometer | WET Labs C-Star | CST-422PR | 1/2/01 | collect |
| Magnetometer | EG&G G-866 |  |  | not collect |
| Gravimeter | Lacoste & Romberg Gravity Meter |  |  | collect |
| Bathymetry | Simrad EK500 | 3001 | 11/1/95 | collect |
| Bathymetry | Bathy 2000 |  |  | collect |

## NBP0104 CTD Sensors:

| Sensor | Description | Serial Number | Cal. Date |
| --- | --- | --- | --- |
| CTD Fish | SeaBird model SBE 9+ | 94857-0232 |  |
| Pressure Sensor | Digiquartz 410K-105 | 43528 | 2/13/01 |
| CTD Deck Unit | SeaBird model SBE 11+ | 11P7536-0317 |  |
| Primary Temperature Sensor | SeaBird model 3plus 6800m | 03P2168 | 8/20/99 |
| Secondary Temperature Sensor | SeaBird model 3plus | 03P2186 | 7/20/00 |
| Primary Conductivity Sensor | SeaBird model 4-02/0 | 041143 | 9/19/00 |
| Secondary Conductivity Sensor | SeaBird model 4C | 040924 |  |
| Dissolved Oxygen Sensor | SeaBird model 13-02-B | 130295 | 7/21/00 |
| Fluorometer | Chelsea model Mk III Aquatracka | 088080 | 7/16/01 |

## NBP0104 MOCNESS Sensors:

| Sensor | Description | Serial Number | Cal. Date |
| --- | --- | --- | --- |
| 1 m2 MOCNESS |  |  |  |
| Conductivity | Sea-Bird 4C | 42067 | 2/13/01 |
| MOC. Electronics/Press. Sensor | B.E.S.S. 16Bit | 0148 | 2/24/01 |
| Optical Plankton Counter | Focal Technologies OPC-1T | TOW031 | 8/1/96? |
| Temperature | Sea-Bird 3+ | 3P2438 | 2/14/01 |
| Transmissometer | WET Labs C-Star | CST-439DR | 2/28/01 |
| Fluorometer | WET Labs AFL | AFL-016D | 2/21/01 |
| 10 m2 MOCNESS |  |  |  |
| MOC. Electronics/Press. Sensor | B.E.S.S. 16Bit | 0149 | 2/24/01 |
| Conductivity | Sea-Bird 4C 6800m | 41852 | 2/13/01 |
| Temperature | Sea-Bird 3plus 6800m | 03P2308 | 2/13/01 |

# Acquisition Problems and Events

This section lists all known problems with acquisition during this cruise including instrument failures, data acquisition system failures and any other factor affecting this data set. The format is jjj:hh:mm (jjj is julian day, hh is hour, and mm is minute).  All times are in GMT.

| Start | End | Description |
| --- | --- | --- |
| 203: |  | Start of Nav and Met data collection |
| 204:13:00 |  | Start of TSG and ADCP data collection |
| 206:04:08 |  | Outside 200-mi limit |
| 207:14:30 | 208:02:00 | Air Temp. reading failed. Cause was a misformatting of output string from RM Young translator. Output format has been modified. |
| 207:14:30 | 208:02:30 | Rmeote and Intenral Seawater temperature readings were non-correleated. Cause was a mixing in of sea wtaer form another intake system in between the two temp. sensors. |
| 217:02:00 | 218:03:00 | Rel. Humidity sesnor had frosted over giving readings > 100% |
| 241: |  | 200-mi limit, end archive of geophysical data |
| 242: |  | End of permit for ADPC/TSG data collection |

# Calibrations

The file instrument.coef contains the calibration factors for the shipboard instruments.  This file is used by the RVDAS processing software in calculations of the using the raw data.

## TSG CALIBRATION FILES

### Conductivity Sensor

### Temperature

### Remote Temperature

### Transmissometer

## Meteorology System

### Windbirds

Mast PAR

### PSP

### PIR

### GUV

## CTD

### Pressure Sensor

### Temperature Sensors

### Conductivity Sensors

### Dissolved Oxygen Sensor

### Fluorometer

### CTD PAR

### CTD Transmissometer

## 10 m2 MOCNESS

### Pressure Sensor

### Temperature Sensor

### Conductivity Sensor

## 1 m2 MOCNESS

### Pressure Sensor

### Temperature Sensor

### Conductivity

### Transmissometer

### Fluorometer

### Optical Plankton Counter